# APPENDIX I

# Scenario Fuel Forecast

Prepared For:

Southern Company Services

Scenario Fuel Forecast Documentation – Budget 2013

Prepared By: Ira Shavel, Chris Goncalves, Chris Ross, Barclay Gibbs, Yingxia Yang, Li Li

Charles River Associates

1201 F Street, NW, Suite 700

Washington, DC 20004

Confidential - November 2012

Table of Contents

1.	INTRODUCTION	1

2.	Overview of FUEL and CO2 View DEVELOPMENT	2

2.1.	Short-Term and Long-Term Fuel Price Forecasting	2

2.2.	Key Drivers	3

2.2.1.	Limitations on Electric Generation Baseload Capacity Additions	3

2.2.2.	Domestic Natural Gas Supply, Particularly From Shale	4

2.2.3.	Expected Environmental Regulations	5

3.	Natural Gas outlook	7

3.1.	Key Market Drivers	7

3.1.1.	Shale Gas Supply	9

3.1.2.	LNG Trade	13

3.1.3.	Drilling Rates and Well Productivity	13

3.1.4.	Policies Governing Shale Drilling Access and Costs	15

3.1.5.	Demand	16

3.1.6.	Baseload Capacity Addition Limits	17

3.2.	Fuel Price Views	19

3.2.1.	Development of the Budget 2013 Fuel Price Views	21

3.2.2.	Low Fuel Price View	23

3.2.3.	Moderate Fuel Price View	25

3.2.4.	High Fuel Price View	27

3.3.	Gas Price Volatility	29

3.4.	Comparison of Budget 2013 to Budget 2012	30

3.5.	Impact of CO2 Stringency on Natural Gas Prices	31

4.	CRUDE OIL OUTLOOK	34

4.1.	Key Market Drivers	36

4.1.1.	Supply	36

4.1.2.	Demand	37

4.2.	Crude Oil Prices	38

5.	Coal OUTLOOK	40

5.1.	Key Market Drivers	40

5.1.1.	Outlook for Coal Capacity	40

5.1.2.	Regulatory Framework for Coal Capacity	41

5.1.3.	Retirements of Coal-fired Generators	48

5.2.	Outlook for Major Coal Basins	49

5.2.1.	Fuel Switching Options – Central Appalachian to Illinois Basin or PRB	53

5.2.2.	Mine Permitting Challenges	54

5.3.	Coal Characteristics and Mine-mouth Coal Prices	55

5.4.	Comparison of Budget 2013 to Budget 2012	57

5.5.	Impact of CO2 Prices on Mine-mouth Coal Prices	59

Appendix A Fuel PRICE Forecasts	61

A.1	Natural Gas Prices	61

A.2	Coal Prices	62

Appendix B Fuel Demand and Production FORECASTS	65

B.1	Natural Gas Demand	65

B.2	Natural Gas Production	67

B.3	Coal Demand	69

Appendix C : MRN-NEEM Model	70

Index of Figures and Tables

Figure 1:  Budget 2013 Planning Scenarios	6

Figure 2:  U.S. Marketed Natural Gas Production and Wellhead Prices	7

Figure 3:  U.S. Shale Gas Plays, Lower 48 States	10

Figure 4:  Southwestern Energy’s Improving Well Performance in Marcellus Shale	11

Figure 5:  U.S. Drilling Rig Gas Count	12

Figure 6:  Estimated Range of Full Cycle Costs for Shale Gas (Moderate View)	14

Figure 7:  Electric Sector Natural Gas Use (Quads) - Existing CO2 Policy Views	16

Figure 8:  Sector Natural Gas Consumption (Quads) - Moderate Fuel Price View, Existing CO2 Policy	17

Figure 9:  Interaction of CRA Gas Models	21

Figure 10:  Henry Hub Natural Gas Prices – Budget 2013 Fuel Views (Existing Carbon Policy)	23

Figure 11:  Domestic Gas Production – Low Fuel Price View (Quads)	24

Figure 12:  Low Fuel Price View Estimate Natural Gas Supply Curve in 2020	25

Figure 13:  Domestic Gas Supply – Moderate Fuel Price View (Quads)	26

Figure 14:  Moderate Fuel Price View Estimate Natural Gas Supply Curve in 2020	26

Figure 15:  Domestic Gas Production – High Fuel Price View (Quads)	28

Figure 16:  High Fuel Price View Estimate Natural Gas Supply Curve in 2020	28

Figure 17:  Historical Henry Hub Natural Gas Prices	29

Figure 18:  Comparison of Budget 2013 and Budget 2012 Natural Gas Price Forecasts, Existing CO2	30

Figure 19:  Comparison of Budget 2013 and Budget 2012 Natural Gas Consumption Forecasts	31

Figure 20:  Comparison of LNG and Crude Oil Pricing	35

Figure 21:  Historical Coal Capacity Additions and Potential Future Additions	41

Figure 22:  Forecasted Cumulative Coal Retirements through 2022 by Scenario	49

Figure 23:  Recent Coal Prices ($/ton)	50

Figure 24:  Coal Demand (Millions of Short Tons) – Historical and Moderate Fuel Price, Existing CO2 Policy Scenario	54

Figure 25:  Thermal Coal Demand Comparison – Budget 2012 and Budget 2013 Forecasts, Moderate Fuel Price, Existing CO2 Policy Scenario	58

Figure 26:  Coal Price Comparison – Budget 2012 and Budget 2013 Forecasts, Moderate Fuel Price, Existing CO2 Policy Scenario	59

Figure 27:  Coal-Fired Generation Responding to CO2 Prices – Moderate Fuel Price View	60

Figure 28:  Sector Natural Gas Consumption (Quads) – Low Fuel Price, Existing CO2 Policy Scenario	66

Figure 29:  Sector Natural Gas Consumption (Quads) – High Fuel Price, Existing CO2 Policy Scenario	67

Figure 30:  Map of MRN-NEEM Regions	70

Table 1:  Limits on New Baseload Generating Capacity (GW)	18

Table 2:  Variations in Fuel Price Views	20

Table 3:  Percentage Changes in Natural Gas Demand (Electric and Non-Electric Sectors) Compared to the Existing CO2 Policy View with Addition of Price on CO2 – Moderate Fuel Price View	31

Table 4:  CRA Crude Oil Price Projections	34

Table 5:  Crude Oil Prices for All Existing CO2 Policy Views (2011$/bbl)	38

Table 6:  Crude Oil Prices Comparison for Existing CO2 Policy Views (2011$/bbl)	39

Table 7:  Comparison of Modeled Environmental Regulations	42

Table 8:  CAIR SO2 and Annual NOX Budgets for the Southern Company Territory States	44

Table 9:  Coal Characteristics	56

Table 10:  Mine-mouth Coal Prices (2011$/ton) - Moderate Fuel Price, Existing CO2 Policy Scenario	57

Table 11:  Henry Hub Natural Gas Prices (2011$/MMBtu) – Existing CO2 Policy	61

Table 12:  Henry Hub Natural Gas Prices (2011$/MMBtu) – Moderate CO2 Policy	61

Table 13:  Henry Hub Natural Gas Prices (2011$/MMBtu) – Substantial CO2 Policy	61

Table 14:  Mine-mouth Coal Prices (2011$/ton) – Low Fuel Price, Existing CO2 Policy Scenario	62

Table 15:  Mine-mouth Coal Prices (2011$/ton) – Moderate Fuel Price, Existing CO2 Policy Scenario	62

Table 16:  Mine-mouth Coal Prices (2011$/ton) – High Fuel Price, Existing CO2 Policy Scenario	62

Table 17:  Mine-mouth Coal Prices (2011$/ton) – Low Fuel Price, Moderate CO2 Policy Scenario	62

Table 18:  Mine-mouth Coal Prices (2011$/ton) – Moderate Fuel Price, Moderate CO2 Policy Scenario	63

Table 19:  Mine-mouth Coal Prices (2011$/ton) – High Fuel Price, Moderate CO2 Policy Scenario	63

Table 20:  Mine-mouth Coal Prices (2011$/ton) – Low Fuel Price, Substantial CO2 Policy Scenario	63

Table 21:  Mine-mouth Coal Prices (2011$/ton) – Moderate Fuel Price, Substantial CO2 Policy Scenario	63

Table 22:  Mine-mouth Coal Prices (2011$/ton) – High Fuel Price, Substantial CO2 Policy Scenario	64

Table 23:  Electric Sector Natural Gas Demand (Quads) – Existing CO2 Policy	65

Table 24:  Electric Sector Natural Gas Demand (Quads) – Moderate CO2 Policy	65

Table 25:  Electric Sector Natural Gas Demand (Quads) – Substantial CO2 Policy	65

Table 26:  Natural Gas Production by Source - Low Fuel Price, Existing CO2 Policy Scenario	67

Table 27:  Natural Gas Production by Source - Moderate Fuel Price, Existing CO2 Policy Scenario	68

Table 28:  Natural Gas Production by Source - High Fuel Price, Existing CO2 Policy Scenario	68

Table 29:  Coal Demand (Million Tons) - Low Fuel Price, Existing CO2 Policy Scenario	69

Table 30:  Coal Demand (Million Tons) - Moderate Fuel Price, Existing CO2 Policy Scenario	69

Table 31:  Coal Demand (Million Tons) - High Fuel Price, Existing CO2 Policy Scenario	69

Disclaimer

All references to views of Charles River Associates and CRA represent views of the authors and do not necessarily represent views of all representatives of CRA.

# INTRODUCTION

This report describes Charles River Associates’ (CRA’s) development of fuel price forecasts for Southern Company’s Budget 2013 planning scenarios.  CRA has produced similar reports associated with the Company's fuel price forecasts for Budget 2010, Budget 2011 and Budget 2012.

The Budget 2013 scenarios share many common assumptions.  They are differentiated, however, by three different views on the future evolution of natural gas supply and demand and by three different views on the future evolution of greenhouse gas (“GHG”) emissions control policy.

CRA’s MRN-NEEM national, multi-sector, energy-economy model, with support from other CRA models, was used to generate integrated scenario results for natural gas and coal prices for the period 2017 through 2052, in five-year increments.

The integrated modeling approach makes it possible to develop forecasts for natural gas, petroleum and coal prices that are internally consistent with each other and with many other variables and feedbacks involving economic growth, electricity consumption and output in many sectors and regions.  The integrated scenario approach takes a set of assumptions about market fundamentals and then solves for the prices that make quantity supplied equal quantity demanded in all these related markets.  In addition, the integrated approach brings out interactions among the different markets and reveals relationships among the fuels that are implied by fundamental factors, such as competition between coal and natural gas as a generation fuel.

The modeling process began with a calibration of the MRN-NEEM model to the most recent Energy Information Agency (EIA) Annual Energy Outlook, in this case the AEO2012 (Early Release Reference Case).  The EIA Early Release Reference Case assumes a continuation of environmental policies in place (finalized not necessarily implemented) at the time of the forecast, while the CRA analyses assume that proposed rules that the United States Environmental Protection Agency (EPA) has yet to finalize (or have finalized since EIA performed its Early Release Reference Case forecast) will come into effect.  As a result, the CRA forecasts anticipate a larger decrease in the coal plant fleet size than EIA does.  These differences from AEO2012 are reflected in the CRA integrated modeling.  It is also noteworthy that the underlying gas supply and demand picture has (again) changed from AEO2011 (used to calibrate the Budget 2012) to AEO2012 (used to calibrate for the Budget 2013 analysis).  The supply picture for shale gas has become more optimistic.

This report describes the models and methods used to develop the fuel forecasts as well as the major underlying assumptions and justifications for these scenarios.

# Overview of FUEL and CO2 View DEVELOPMENT

Three different fuel views were developed with differing views on future supply and demand.  In addition, the following assumptions were used:

Moody’s Analytics forecast of Gross Domestic Product (GDP) growth and Southern’s assumption about the relationship between growth in electricity demand and growth in GDP;

Costs, performance and limits on annual builds and cumulative capacity additions of different electric generating technologies;

No new coal units without carbon capture and storage (CCS) will be built, except for identified units for which construction has already begun;

Increasingly stringent environmental regulations with respect to air emissions (SO2, NOX and Hazardous Air Pollutants, or HAPs), coal combustion residuals, water usage at coal-fired generating facilities and renewable electricity mandates.

Since no model can accurately predict specific future outcomes, it is important to include a range of scenarios that encompass the range of likely outcomes for important factors that affect the planning process.

For purposes of these forecasts, three fuel views were selected.  These views were developed to encompass the range of potential fuel price outcomes.  These were then paired with each of three CO2 price paths - $0, $10, $20 (in REDACTED dollars) per metric ton of CO2, increasing at REDACTED% in real dollar terms.  Hereafter, the three CO2 price paths will be referred to as Existing, Moderate and Substantial CO2 policy views, respectively.  Pricing of CO2 in the Moderate policy view begins in 2017 and in 2020 for the Substantial policy view.  The combination of the three fuel price views and the three CO2 policy views produces nine integrated scenario forecasts of fuel prices.

## Short-Term and Long-Term Fuel Price Forecasting

The models used in this analysis are focused on long-term results.  To account for this, and to utilize high-quality data on near-term expected prices, a blending approach is utilized.  Under this blending approach, Southern Company uses near-term data from futures markets and similar sources in the initial years (current year plus next two years) of their internal modeling efforts.  To bring together the near-term data with the long-term forecasted results Southern Company interpolates results between the short and long terms, and then from 2017 and after the results from this report are utilized.

Short-term forecasts are updated monthly as part of Southern Company’s fuel budgeting process and marginal pricing dispatch procedures.  The short-term forecasts are overseen by Southern Company’s Fuel Services department.  The long-term forecasts described in this report are developed in early spring of each year for use in system planning activities, business case analyses and decision making.  The development and documentation of the long-term forecasts is a collaborative effort between CRA and Southern Company and occurs as a part of the planning scenario development process under the guidance and direction of the Planning Coordination Team (PCT).  Once the updated long-term forecasts are developed, they are integrated with the short-term forecasts.

The short-term and long-term forecasts are separated for two reasons.  The first is that the short-term forecasts are used for company activities such as regulatory fuel filings and operational budgets, which must reflect current laws that are in place.  The long-term forecasts reflect a range of projections that take into account probable laws and regulations that are not yet in effect, yet represent Southern Company's expectation of a reasonable range of those future potential laws.  Secondly, the short-term forecasts are based on actual market prices, which reflect current market conditions and market assumptions of the near future but which do not always reflect regulatory, legislative or other impacts assumed in the modeling of the multiple long-term scenario forecasts.

Further, the CRA natural gas model includes detailed projections of natural gas resources, cost curves and demand sectors through 2030.  Beyond 2030, such a detailed analysis is not practical due to the very high uncertainty of each of these factors, and prices were set based on scenarios of oil to natural gas ratios that were consistent with the gas model price trends from 2020-2030. In the low fuel price view, the 2030 oil to gas ratio was REDACTED and declined to REDACTED by 2050.  In the moderate fuel price view, the 2030 ratio of REDACTED declined to REDACTED by 2050.  In the high fuel price view, the 2030 ratio of REDACTED declined to REDACTED by 2035 and stabilized at that level through 2050.

## Key Drivers

### Limitations on Electric Generation Baseload Capacity Additions

There are many new generating capacity options in the electric sector, including coal, natural gas, nuclear capacity, and renewables.

While there are many different options, political and economic realities suggest that there are limitations on the quantity and timing of these potential new generating additions.  To reflect these realities, limits on several types of generating capacities have been applied:

New coal units without CCS are limited to those projects that are currently under construction.

CCS technology on new coal units and new nuclear generating units are assumed unavailable until REDACTED.  New nuclear generating capacity may be limited by several factors including manufacturing capacity, the ability to finance, and delays or objections to operating or obtaining environmental permits due to public perceptions regarding nuclear technology.  These two technologies, which are low carbon baseload resources that are likely to compete against each other, are jointly limited in the model.

New additions of intermittent resources such as wind and solar are limited to ensure a reliable supply system.  Other renewables such as biomass, landfill gas and geothermal have regional limits based on available resources such as feedstocks and geologic suitability.

All of these limitations have consequences for the fuel price views.  Natural gas prices are particularly sensitive to these assumptions as natural gas becomes a transition fuel, if other generating technologies are limited.

### Domestic Natural Gas Supply, Particularly From Shale

The two most important factors in developing natural gas views are the magnitude and timing of demand growth (including from power generation, industrial, transportation, and LNG exports), and the cost, magnitude and accessibility of shale resources.

Natural gas demand expectations vary widely as a function of expectations for:

- The impacts of economic growth and environmental policy on gas consumption in the power sector (especially relative to coal);

- The oil-gas price spread and its impact on the resuscitation and development of natural gas liquids (NGL) and natural gas-intensive industrial facilities such as in the petrochemical, agribusiness, and metals industries;

- The oil-gas price spread and energy policy impacts on the amount of natural gas consumed in the large and medium fleet vehicle sector (as LNG and compressed natural gas, or CNG).

The volume and cost of producible shale gas resources is equally important and equally subject to a wide array of expectations.  If shale gas production continues to prove plentiful (and relatively inexpensive), then the U.S. will be self-sufficient in natural gas supply, and prices will be set by the interplay of shale gas production economics and demand in the electric power, industrial and transportation sectors, with only a modest upwards drift in prices.

If shale production does not materialize on the scale and at the cost many now expect, this factor combined with growing demand and LNG exports could eventually produce stronger upward price impacts.

Thus, there could be a discontinuity between natural gas views in which domestic supply can satisfy domestic demand at sustained low prices well below the world price of LNG, and views in which high demand growth and shale production constraints drive prices higher to levels that narrow differentials with LNG prices and thus begin to impact the export thesis for LNG over the long term.  During the period between preparing the Budget 2012 fuel price views and starting the Budget 2013 analysis, there was further confirmation of the size of the shale gas resource, particularly shale gas from “liquids-rich” plays.  The net result is that Budget 2013 natural gas prices are lower for all three fuel price views.  LNG plays a role only in the High price view and then only in the long term.

Also new for Budget 2013 is the possibility that demand for domestic natural gas might increase due to LNG exports, as well as an industrial resurgence and growth in transportation use of natural gas.  We assume increasing levels of new demand going from the Low price fuel view to the High price fuel view.

### Expected Environmental Regulations

A range of new environmental regulations is on the horizon.  These regulations are expected to cover air emissions (such as SO2, NOX and HAPs), GHG emissions like CO2, water usage, coal combustion residuals and mandates on renewable energy use (particularly in the electric sector).

These regulations have several impacts on the operations of existing electric generators and the selection of the types of new generating capacity to be built.  As many of these regulations place a significantly higher burden on existing coal-fired generators, these units face greater risks of retirement or lower dispatch.  Coal retirements and/or lower dispatch have implications for the demand and pricing of coal and natural gas, the latter of which can substitute for coal in the electric sector, especially in the near-term.

The range of CO2 price views considered in this year’s forecasts has starting prices of $0 to $20 per metric ton of CO2 (in REDACTED dollars), and then rises at REDACTED% annually plus inflation.  By analyzing a wide range of CO2 prices, including an Existing CO2 policy case where the price of CO2 is zero, a better understanding of the potential outcomes from carbon policy can be gained.

The form and stringency that CO2 emissions control will take in electricity generation is unclear.  At the beginning of 2011, CO2 became a pollutant controlled under the Clean Air Act (“CAA”).  On April 13, 2012, EPA proposed New Source Performance Standards (NSPS) for GHGs for new electric generating units. The proposed rule would not apply to existing generators or those for which construction begins during the subsequent 12 months.  The proposed rule would set a 1,000 lbs CO2 per MWh (gross) output-based standard, with an option to demonstrate compliance over 30 years.  However, EPA’s language in the proposed rule and past relevant CAA regulatory patterns foreshadows possible, future regulation of CO2 from existing generators.

The current Administration has reaffirmed a commitment to reducing U.S. GHG emissions and the Edison Electric Institute remains committed to reducing electricity GHG emissions to 80% below 2005 levels by 2050.  At the end of the previous decade, the U.S. Congress considered many proposals to reduce GHG emissions. One such proposal got as far as passing the U.S. House of Representatives in 2009. Congress has also considered, but not passed, legislation to preclude regulation of GHGs under the existing CAA.  In the absence of such legislation, the U.S. EPA is proceeding to develop and implement GHG regulations under the existing CAA (and could propose regulations to reduce GHG emissions from existing sources). The U.S. Supreme Court has affirmed that the U.S. EPA has the authority to proceed (Massachusetts v. EPA), and legal challenges to EPA’s particular approach were recently rejected by the courts.  It should be noted that the impacts of a regulatory approach to GHG policy could potentially have much larger impacts on electricity generation than a legislative approach.  A regulatory approach may provide less flexibility and may be more costly than a legislative approach (for a given level of abatement).  Quantifying these costs in advance of final regulation is difficult, given the numerous regulatory approaches EPA may take under the CAA.

In the presence of uncertainty in the form and stringency of future CO2 emissions, this analysis has considered three different carbon price views that span a wide range of possible, future CO2 stringency:

- The least stringent view respects Existing CO2 requirements, including a view that there will be no new coal units without CCS (except for the units currently under construction);

- The Moderate CO2 stringency view adds an economy-wide price on CO2 emissions that begins at $10 per metric ton (REDACTED$) in 2017 and rises at REDACTED% above inflation;

- The Substantial CO2 stringency view has an economy-wide price on CO2 emissions that begins at $20 per metric ton (REDACTED$) in 2020 and rises at REDACTED% above inflation.

Both the Moderate and the Substantial carbon price views are coupled with the assumption that new natural gas combined-cycle power generators will be required to have CCS technology beginning in 2030.  CRA believes that this range of views reasonably spans the current uncertainty in long-term CO2 pricing relevant for electricity system planning.  Political and economic developments over the last few years cause us to believe that our Substantial CO2 policy view is likely a reasonable high bound (and a higher price is not necessary to model).

Summarizing, we have analyzed the nine scenarios identified in

Figure 1.

Figure 1:  Budget 2013 Planning Scenarios

|  | CO2 Views |  |  |
| --- | --- | --- | --- |
|  | Existing ($0/MTCO2) | Moderate ($10+/MTCO2)a Starts in 2017 | Substantial ($20+/MTCO2)a Starts in 2020 |
| Fuel Views |  |  |  |

a  In REDACTED dollars, rising at REDACTED% above inflation

# REMAINDER OF REPORT REDACTED IN ITS ENTIRETY

The exception for new coal units with CCS is the Kemper County Plant Ratcliffe IGCC plant in Mississippi.  Exceptions for new nuclear are Vogtle 3 and 4 in Georgia and Watts Bar 2 in Tennessee.

In June 2012, the U.S. Court of Appeals for the District of Columbia unanimously rejected four challenges to EPA GHG rules.  The Court did the following:  1) upheld the “endangerment finding,” 2) upheld the “tailpipe rule” (regulating GHG emissions from light duty motor vehicles), 3) confirmed that the tailpipe rule implies that GHGs are regulated under Prevention of Significant Deterioration (PSD) and Title V permitting, and 4) concluded that the challenges to EPA’s Timing Rule and Tailoring Rule were not justified.