GEORGIA POWER COMPANY

BUDGET 2019

LOAD AND ENERGY FORECAST

2019 TO 2038

Prepared by:

Georgia Power Company

Georgia Power Company

Budget 2019 Load and Energy Forecast

Table of Contents

Table of Contents

1.0	EXECUTIVE SUMMARY OVERVIEW	1

1.1	PEAK DEMAND, TERRITORIAL SUPPLY, AND TERRITORIAL REQUIREMENTS	3

1.2	RETAIL ENERGY SALES	5

1.2.1 Residential	5

1.2.2 Commercial	6

1.2.3 Industrial	7

1.2.4 Other Retail: Governmental Lighting, MARTA	8

1.3	ECONOMICS	9

1.4	FORECAST ASSUMPTIONS AND METHODS	10

1.5	DEMAND SIDE PROGRAMS	11

1.6	SENSITIVITIES AND SCENARIOS	12

2.0	ANNUAL SUMMARY	13

2.1	MONTHLY ENERGY REQUIREMENTS	14

2.2	CUSTOMERS	17

3.0	GENERAL FORECASTING OVERVIEW	18

3.1	ECONOMETRIC REGRESSION	18

3.2	END-USE MODELS	18

3.3	JUDGMENT	19

3.4	TOOLS	19

4.0	SHORT-TERM MODEL DESIGN OVERVIEW	20

4.1	RETAIL SALES SECTORS	20

4.1.1	Residential Sales	20

4.1.2	Commercial Sales	23

4.1.3	Industrial Sales	26

4.1.4	MARTA Sales	28

4.1.5	Governmental Lighting Sales	30

4.2	CUSTOMER FORECASTS	30

4.2.1	Residential Customers	30

4.2.2	Commercial Customers	31

4.2.3	Industrial Customers	32

4.2.4	Governmental Lighting Customers	32

4.3	CALENDAR MONTH FORECAST	33

5.0	LONG-TERM MODEL DESIGN	35

5.1	LONG-TERM ENERGY SALES	35

5.1.1	Residential Long-Term Energy Sales	35

5.1.2	Commercial Long-Term Energy Sales	59

5.1.3	Industrial Long-Term Energy Sales	69

5.2	ANNUAL TO MONTHLY BASED ENERGY FORECASTS	78

5.3	INTEGRATION OF SHORT- AND LONG-TERM ENERGY FORECASTS	79

6.0	PEAK DEMAND FORECAST	80

7.0	SENSITIVITY AND SCENARIO DEVELOPMENT OVERVIEW	94

7.1	SENSITIVITIES	94

7.2	SCENARIOS	97

8.0	FORECAST ASSUMPTIONS OVERVIEW	102

8.1	SHORT-TERM ENERGY MODEL ECONOMIC ASSUMPTIONS	103

8.2	LONG-TERM ENERGY MODEL ECONOMIC ASSUMPTIONS	103

8.2.1	Overview	103

8.2.2	Income per Household	103

8.2.3	Floor Space (Commercial Activity)	104

8.2.4	Industrial Production (Industrial Activity)	104

8.2.5	Demographics	104

8.2.5.1 Persons per Household	104

8.2.5.2 Housing Type Distribution	104

8.2.5.3 Commercial Employment Rate (Commercial Activity)	105

8.3	FUEL PRICE ASSUMPTIONS	105

8.3.1	Forecasts by Fuel Type: Electricity and Natural Gas	105

8.3.2	Substitution of Electricity with Competing Fuels	105

8.4	FORECAST ADJUSTMENTS	106

8.4.1	Demand Side Strategies and Customer Actions	106

8.4.2	Cogeneration	107

8.4.3	Real Time Pricing and Time of Use	107

8.4.4	Electric Vehicles	108

8.4.5	Behind-The-Meter Solar	108

8.5	IMPACTS OF APPLIANCE EFFICIENCY STANDARDS	108

8.6	Budget 2019 Energy Prices and Forecast Adjustments	110

8.7	RESOURCE SIDE DEMAND SIDE OPTIONS	115

9.0	FINAL IRP REQUIREMENT OVERVIEW	116

9.1	DESCRIPTION OF WEATHER NORMALIZATION	116

9.2	REVIEW OF ECONOMIC INPUT AND LOAD PROFILES	117

9.3	ACTUAL & EXPECTED INTERRUPTIBLE DEMAND	155

9.4	EVALUATION OF PREVIOUS FORECASTS	155

9.4.1	General Enhancements	161

9.4.2	Specific Enhancements	162

9.5	ENERGY RESEARCH PLAN	168

9.5.1	Introduction	168

9.5.2	Survey Research	170

9.5.2.1  Regulatory Requirements	170

9.5.2.2  Data Collection and Utilization Plan	170

9.5.3	Load Research	170

9.5.3.1  Regulatory Requirements	170

9.5.3.2  Class Load Research Project	171

9.5.4	Analysis	171

9.5.4.1  Regulatory Requirements	171

9.5.4.2  Analysis Projects	171

10.0 ACRONYM & ABBREVIATION GLOSSARY	173

# 1.0	EXECUTIVE SUMMARY OVERVIEW

The purpose of this report is to summarize the Budget 2019 Load and Energy Forecast (“Budget 2019”).

A twenty-year forecast of energy sales and peak demand was developed to meet the planning needs of Georgia Power Company (“Georgia Power” or the “Company”). Budget 2019 includes the following retail classes: residential; commercial; industrial; Metropolitan Atlanta Rapid Transit Authority (“MARTA”); and governmental lighting. The baseline forecast was started in the spring of 2018 and completed in the fall of 2018.

Both U.S. and Georgia experienced solid economic growth from 2013-2017. Over this period, U.S. Gross Domestic Product (“GDP”) growth averaged 2.3% per year, while Georgia’s economy experienced growth of 3.1% per year. Also during this period, U.S. employment grew by an average rate of 1.8% per year and the unemployment rate fell from 8.0% to 4.1%. In Georgia, employment growth averaged 2.5% per year and the state’s unemployment rate fell from 8.7% to 4.5%. In 2018, the unemployment rate fell below 4% for both the U.S. and Georgia.

Despite solid economic growth in Georgia, Georgia Power’s total retail energy sales have flattened since 2007. Over the 2007-2017 period, weather normalized total energy sales fell at an average annual rate of -0.2% and remain below 2007 levels. The commercial and industrial classes declined over this same period, down an average of -0.2% and -0.8% per year, respectively. The residential class is the only one that experienced modest growth, up an average of 0.2% per year over the past ten years. In more recent years, total retail sales began to grow due to the strength of the economy in Georgia, as evidenced by strong growth in the number of total customers. Between 2013 and 2018, total retail energy sales grew at an average annual rate of 0.5%.

Georgia’s economic growth is expected to continue over the next few years. One factor contributing to growth is that the state is an attractive place to do business. Georgia’s low cost of doing business and low cost of living, the deep pool of knowledge and technical workers coming from its university system, its globally connected airport and transportation infrastructure (e.g. ports, highways), and its business-friendly government policies will continue to attract businesses. Positive demographic trends are another factor driving growth in the state. As businesses continue to relocate and expand in Georgia, the state will experience solid employment growth, which will attract new residents. As a result, population growth will remain above the U.S. average.

Additional businesses and a growing population are expected to provide a boost to energy sales. From 2019-2029, total energy sales are projected to grow at an average annual rate of 0.7%. Residential sales are expected to grow by an average of 1.2% per year over this period as the increase in the number of customers outpaces the reduction in use per customer resulting from energy efficiency. Industrial sales are expected to increase at an average annual rate of 1.0% primarily due to the addition of a very large customer. Sales to the commercial class are expected to remain nearly flat, declining by an average of −0.1% per year due in part to increased energy efficiency. Peak demand is expected to increase at an average rate of 0.5% per year.

A more detailed discussion of customer class energy sales and forecast results is presented below.

# 1.1	PEAK DEMAND, TERRITORIAL SUPPLY, AND TERRITORIAL REQUIREMENTS

A peak demand of 17,985 MW was set on August 9, 2007 for the Georgia Power service territory. Over the past 10 years, Georgia Power’s total peak demand has not surpassed this level. In 2021, weather normal peak demand is expected to be REDACTED MW and then to grow to REDACTED MW in 2029. Figure 1.1-1 shows the comparison of the Budget 2018 and Budget 2019 peak demand forecasts. In Budget 2019 and Budget 2018, peak demand is expected to grow at a compound average annual rate of 0.5% from 2019 to 2029. Over the forecast horizon, Georgia Power is expected to be a summer-peaking utility.

Figure 1.1-1: Peak Demand Forecast

The peak demand forecast for Budget 2019 shown here has been adjusted to reflect the effects of Real Time Pricing (“RTP”) customer response to changes in prices, expected cogeneration, and the impacts of DSM programs.

Figure 1.1-2 shows the expected growth in territorial energy sales over the forecast horizon. In absolute terms, the amount of GWh growth is indicated by the average annual growth rate (“AAGR”). During the historical period from 2007 to 2017, which includes the Great Recession (2007-2009), average growth declined by 374 GWh per year. Budget 2019 anticipates an average growth of REDACTED GWh each year from 2018-2028, while Budget 2018 predicted growth of REDACTED GWh per year.

Figure 1.1-2: Territorial Energy Sales

# 1.2	RETAIL ENERGY SALES

## 1.2.1 Residential

In the short term, 2019-2021, Budget 2019 residential energy sales projections are similar to those in Budget 2018. In the longer run, Budget 2019 grows slightly slower and remains below Budget 2018.  Figure 1.2.1-1 shows the comparison of Budget 2019 with the Budget 2018 residential energy forecast.  In the later years of the forecast, Budget 2019 residential energy forecast falls below that of Budget 2018 largely due to a lower forecast of electric vehicles.

The Budget 2019 residential customer forecast is not much different than Budget 2018. Both forecasts are driven by recent strong growth in customer additions in the short-term and by household growth in the long-term.

Figure 1.2.1-1 Residential Energy Forecast

## 1.2.2 Commercial

From 2007-2017, average annual growth in commercial sales has been relatively flat, and sales remained below their pre-recession levels. Both Budget 2019 and Budget 2018 project slight annual declines over the next several years before growth picks up in the outer years of the forecast. This can be seen in figure 1.2.2-1 below.

The modest decline in sales through 2027 is driven primarily by improvements in energy efficiency. In the latter years of the forecast, growth picks up due to expected growth in commercial square footage and electric vehicles. Budget 2019 is lower than Budget 2018 due to efficient lighting and other technology updates in the commercial long-term forecast model and a lower electric vehicle forecast.

Figure 1.2.2-1: Commercial Energy Forecast

## 1.2.3 Industrial

The industrial sectors in both the U.S. and Georgia were hit especially hard during the 2007-2009 recession. Since 2007, Georgia Power’s industrial energy sales have fallen by an average of 0.8% per year and have still not returned to their pre-recession levels. Figure 1.2.3-1 shows a comparison of Budget 2019 and Budget 2018. Budget 2019 is lower due primarily to a change in the timing of a large new customer coming on-line. In the longer term, industrial production is expected to grow at a slightly faster pace than in Budget 2018.

Figure 1.2.3-1: Industrial Energy Forecast

## 1.2.4 Other Retail: Governmental Lighting, MARTA

The Budget 2019 governmental lighting forecast is somewhat lower than Budget 2018 in the near term. Figure 1.2.4-1 shows a comparison of the two forecasts for governmental lighting. Governmental lighting sales have declined significantly since 2014 as traditional street lights are changed to LED lights, which use much less energy. Budget 2019 assumes that planned changeouts will be completed in 2021, compared to the 2020 estimated completion in Budget 2018.

Figure 1.2.4-1: Governmental Lighting Energy Forecast

The Budget 2019 MARTA energy sales forecast is about the same as Budget 2018. Since MARTA creased the frequency of its trains in the second half of 2014, annual energy sales have been relatively flat. Figure 1.2.4-2 shows a comparison of the two forecasts for MARTA.

Figure 1.2.4-2: MARTA Energy Forecast

# 1.3	ECONOMICS

Georgia’s economy is performing well. In 2018, the state set records for the total number of jobs, total number of people employed, and total labor force. Employment statistics through November 2018 show that jobs in Georgia increased 2.2% compared to 2017, a rate that is ahead of the nation’s 1.7% rise over the same period. In November, Georgia’s 3.5% unemployment rate was slightly below the 3.7% rate for the country. Construction, professional and business services, education and health services, trade and transportation, and leisure and hospitality are the sectors providing the majority of the state’s job growth in 2018.

Georgia remains an attractive place to do business. The state’s business climate has been ranked number one in the nation by Site Selection Magazine for six straight years, while Area Development has ranked Georgia the top state for business for five consecutive years. As a result, companies, both large and small, continue to relocate and expand operations in the state. Corporate relocations will help fuel growth in employment, population and incomes. Over the next ten years, total non-agricultural employment gains are expected to average REDACTED% per year, compared to REDACTED% growth for the U.S.

The state’s population growth outpaced that of the U.S. In 2017, Georgia’s population grew 1.1%, compared to 0.7% for the nation as a whole. Since 2010, Georgia added 740,000 residents, and is now the 8th most populous state in the nation. Georgia’s projected employment outlook and favorable living costs will support in-migration, which in turn drives population growth and household formation. In the forecast, Georgia’s population is expected to increase an average of REDACTED% per year between 2018 and 2028, while the nation’s population is expected to increase by REDACTED% per year. From 2007-2017, housing starts averaged 37,000 per year in Georgia. Over the next 10 years, new home construction is projected to increase more strongly, with housing starts averaging over REDACTED.

Increases in Georgia’s population, households and business formations are expected to provide a firm foundation for robust long-term economic growth in the state.

# 1.4	FORECAST ASSUMPTIONS AND METHODS

The assumptions underlying Budget 2019 were developed through the joint effort of Georgia Power and Southern Company Services (“SCS”). The forecast was developed through careful consideration and methodical examination of key demographic and economic variables that historically have been significant indicators of energy consumption. Major assumptions include the economic outlook for the U.S. and Georgia, energy prices, and market profiles for class end uses.

The economic forecast provides a description of the economy for the next 20 years and includes many elements of the economy such as gross product, population, employment, commercial building square footage, and industrial production. The economic and demographic forecasts for Budget 2019 were obtained from IHS Markit, a national provider of economic data and forecasts. Budget 2019 is the first forecast using IHS Markit rather than Moody’s Analytics.

The models used to produce both the short- and long-term energy forecasts include a variety of economic and demographic variables as drivers of energy use. Weather, income, employment, historical load data, and industry standards for electrical equipment are among the variables used in the forecasting models. “Normal” weather is defined as the average of Cooling Degree Hours (“CDH”) and Heating Degree Hours (“HDH”) from 1980-2017.

Short-term energy projections, with the exception of governmental lighting, are based on linear regression models developed for the various energy classes. The governmental lighting forecast is developed using information from Georgia Power field personnel. The details of these regression models can be found in Section 4.

The long-term forecast models are end-use models. Budget 2019 uses the Load Management Analysis and Planning (“LoadMAP”) model to produce the long-term residential, commercial, and industrial forecasts. The LoadMAP tool is discussed in greater detail in Section 5.

The long-term MARTA and governmental lighting forecasts are not based on end-use models. The MARTA forecast uses the linear regression model developed for the short-term forecast, while the governmental lighting forecast is based on information from Georgia Power field personnel.

The results of the short-term and long-term models are integrated into a unified forecast. In Budget 2019, the short-term forecast results were used for the years 2019 through 2021 and the long-term results from 2022 to 2038. Additional information on methodology can be found in Section 3.

Budget 2019 uses the Peak Demand Model (“PDM”) to predict Georgia Power’s weather-normal peak demands. The methodology and assumptions used in the PDM tool are discussed in greater detail in Section 6.

# 1.5	DEMAND SIDE PROGRAMS

Georgia Power offers energy efficiency programs to promote energy savings to customers. These programs have existed for several years, and the benefits from these programs are embedded in Georgia Power’s historical loads and energy sales. The forecasts produced by using historical energy data, therefore, have an embedded effect propagated throughout the forecast. Other conservation measures undertaken by customers, whether explicitly induced by a marketing program or proactively undertaken by a customer for other reasons, will also become embedded in the historical energy data and will be implicitly propagated throughout the forecast. New or ongoing Georgia Power programs that are not fully reflected in historical data are incorporated into the forecast as external adjustments.

An additional type of demand-side impact considered in Budget 2019 is the adopted efficiency standards in the end-use models used for the long-term forecast. These efficiency standards represent the continuing trend of increasing end-use efficiency driven in part by programs such as ENERGY STAR®.

# 1.6	SENSITIVITIES AND SCENARIOS

Budget 2019 is a base case forecast, using as its foundation the most probable economic scenario and current regulatory environment. Nine scenarios were developed to evaluate the impacts of possible outcomes under different regulatory environments with respect to various carbon and fuel price combinations.

It is also important to understand the possible impacts of variations in the description of the factors (variables) that are used in the forecast and their impact on the forecast results. Consequently, five forecast sensitivities have also been run: 1) high economic growth; 2) low economic growth; 3) no load growth; 4) load growth with no DSM growth; and 5) load growth with aggressive DSM growth. The economic sensitivities were driven by variations in the economic drivers of the forecast provided by IHS Markit. Section 7 provides the details of the various scenarios and sensitivities.

# 2.0	ANNUAL SUMMARY

Attachment 2.0-1: Budget 2019 Forecast Annual Summary

# 2.1	MONTHLY ENERGY REQUIREMENTS

Attachment 2.1-1: Budget 2019 Monthly Energy Requirements by Class of Service – Forecast

Attachment 2.1-1: Budget 2019 Monthly Energy Requirements by Class of Service – Forecast

Attachment 2.1-2: Monthly Energy Requirements by Class of Service – Historical

# 2.2	CUSTOMERS

Attachment 2.2-1: Budget 2019 Historical and Forecast Customers (Annual Average)

# 3.0	GENERAL FORECASTING OVERVIEW

This section provides an overview of the modeling methods used to produce Budget 2019 for Georgia Power. The forecast uses a variety of statistical techniques and other methods. The availability of data, characteristics of the market, and uses of the forecast all help determine which method will be used for the particular parts of the forecast.

Budget 2019 uses three primary methods:

1)	Econometric regression;

2)	End-use models; and

3)	Judgment.

# 3.1	ECONOMETRIC REGRESSION

Econometric regression is a statistical technique with which model parameters are estimated from time series, cross sectional, or a combination of time series and cross sectional (pooled) data. Basic classical regression assumptions apply to the models. These models are well suited for modeling the relationships between energy consumption and explanatory variables such as economics, weather, and changes in behavior such as increases in the use of electronic devices in the home. Georgia Power’s short-term (2019-2021) forecasts utilize econometric models.

# 3.2	END-USE MODELS

End-use forecasting models are used to represent the elements of energy use in fine detail. End-use models estimate residential energy use by looking at appliance saturation and penetration surveys and then determining aggregate class usage by adding up all the elements of energy consumption across households. Each major energy-consuming activity – refrigeration, space heating, and the like – is identified, and the corresponding energy consumption is specified. This approach has a strong intuitive appeal because energy consumption is clearly associated with appliance use. The models can also produce energy sales forecasts of end-use equipment with a wide range of efficiency and usage sensitivities with respect to fuel prices, income levels, and other factors.

The detailed nature of end-use models is their major advantage relative to econometric models, but the detailed input requirements are also the major source of their limitations. These models require extensive information about projected appliance market penetration and usage rates. It is a challenge to produce such projections that are sensitive to changes in energy prices, household incomes, and lifestyles.

The end-use models used by Georgia Power for its long-term energy forecasts combine the benefits of end-use detail and class segmentation to estimate future energy consumption. These models utilize econometric techniques to model consumer choices of end uses based on economic principles.

# 3.3	JUDGMENT

Even the best modeling techniques need to be augmented by insights gained through relationships with customers and through discussions with marketing personnel. In cases where on-site generation equipment is added at existing sites, an industrial plant closes, or a large new customer is added to the electrical grid, for example, econometric and end-use models may not be able to adequately capture the impact. In order to supplement these modeling tools, some amount of judgment is used to refine the forecast.

# 3.4	TOOLS

The tools used to prepare the energy and peak forecasts include EViews (regression software), LoadMAP, PDM and Microsoft Excel.

# 4.0	SHORT-TERM MODEL DESIGN OVERVIEW

This section describes the quantitative methods used to develop the Georgia Power customer and short-term energy sales forecasts. The forecasts from the short-term energy models cover the period from January 2019 through December 2021. These short-term energy forecasts are then integrated with the long-term results discussed in Section 5. The residential and commercial short-term energy models generate results on a billing month basis, which are then converted to a calendar month basis. Industrial sales are modeled on a calendar month basis. The MARTA and governmental lighting energy forecasts are produced on a billing month basis.

The estimate of the number of customers by class is derived from econometric models that have a twenty-year forecast horizon. The short-term energy forecasts are also developed on a customer class basis and employ a linear regression methodology. Although the drivers vary by customer class, the models include explanatory variables that relate changes in monthly sales or customers to changes in Georgia’s economy, demographic make-up, weather, energy efficiency, and monthly and/or monthly weather interaction binary variables.

The short-term model parameters are estimated with the EViews software package. EViews is a well-established software package used in econometric analysis, forecasting, and simulation. IHS Markit provides the historical and forecast economic data used in the short-term models. When estimating the model parameters, actual HDH and CDH are used. For the forecast period, “normal” weather is assumed. “Normal” weather for Budget 2019 is defined as the average of CDH and HDH from 1980-2017.

# 4.1	RETAIL SALES SECTORS

The retail sector includes the residential, commercial, industrial, MARTA, and governmental lighting classes.

## 4.1.1	Residential Sales

The model of energy sales for the residential class depends on the following factors:

Number of customers

Number of days in the billing cycle

Weather

Energy efficiency

The impact of demographic trends on residential sales are included by modeling energy use with a use-per-customer formulation. The short-term residential energy sales forecast, therefore, depends upon a forecast of residential customers. The residential customer forecast is presented in Section 4.2.1.

Weather has a significant impact on residential energy sales. HDH and CDH are used to measure the impact of weather on monthly residential energy sales. The degree hours are computed as an average of the billing cycles. Residential HDH use a 54°F threshold, and CDH use a 67°F threshold. Degree hours are averaged over ten U.S. Weather Bureau stations, and the temperatures at the weather stations are weighted by the residential energy use in the Georgia Power regions that are mapped to the ten weather stations.

Another consideration is the variation in the length of the billing period. Historical average billing days per month are obtained from the annual meter reading schedules. For Budget 2019, average billing days were based on a fixed meter reading schedule for the January 2019 – January 2020 period.

This impact on sales resulting from the variation in the length of the billing period is addressed by dividing the use-per-customer (dependent) variable and the monthly weather interaction terms by the number of billing days.

A regression model relating residential energy use per customer per billing day to an intercept, twelve monthly weather interaction variables, and an energy efficiency variable was developed for the short-term residential energy forecast. For the monthly weather interaction variables, CDH per billing day and HDH per billing day are interacted with monthly binary variables to better distinguish their impact on the residential class at various times of the year.

The forecast of residential use per customer per billing day is multiplied by the forecasted number of billing days and the forecasted number of customers to get forecasted residential billed energy sales. Table 4.1.1-1 contains a summary of the parameter estimates and statistics for the short-term residential energy model.

Table 4.1.1-1 Residential Use per Customer per Billing Day

Short-Term Model

Budget 2019

Where:

## 4.1.2	Commercial Sales

The model of energy sales for the commercial class depends on the following factors:

Number of customers

Number of days in the billing cycle

Weather

Energy efficiency

Commercial energy sales (MWh) are projected using a linear regression model that relates monthly energy use per customer per billing day to independent variables that capture the impacts of varying weather conditions and energy efficiency.

HDH and CDH are used to measure the impact of weather on monthly commercial energy sales. As with the residential class, HDH and CDH are computed as an average of the billing cycles. HDH are computed with a 51°F threshold and CDH are computed with a 62°F threshold for the commercial class. Degree hours are averaged over ten U.S. Weather Bureau stations, and the temperatures at the weather stations are weighted by the commercial energy use in the Georgia Power regions that are mapped to the ten weather stations.

Similar to the residential model, the impact on sales resulting from the variation in the length of the billing period is addressed by dividing the commercial energy use per customer (dependent) variable and the degree hour seasonal interaction terms by the number of billing days.

The regression model relates monthly commercial energy sales per billing day to an intercept, twelve monthly weather interaction terms that are divided by the number of billing days, and an energy efficiency variable. The monthly weather interaction variables are calculated by multiplying the CDH per billing day and HDH per billing day with monthly binary variables. This helps distinguish the impact of HDH and CDH on the commercial class at various times of the year.

The forecast of commercial energy sales per billing day is multiplied by the forecasted number of billing days and the number of customers to get forecasted commercial billed energy sales. A summary of parameter estimates and statistics for the short-term commercial energy model is found in Table 4.1.2-1.

Table 4.1.2-1: Commercial Use per Customer per Billing Day

Short-Term Model

Budget 2019

Where:

## 4.1.3	Industrial Sales

The model of energy sales for the industrial class depends on the following factors:

Weather

Number of calendar days in the month

Economics

Industrial energy use per calendar day (MWh) is projected using an econometric regression model with independent variables that capture the impacts of varying weather conditions and Georgia economic trends.

While not as weather sensitive as commercial or residential classes, CDH are used to measure the impact of weather on monthly industrial energy sales. CDH are computed as an average of the billing cycles using a 59°F threshold. The CDH are averaged over ten U.S. Weather Bureau stations, and the temperatures at the weather stations are weighted by the industrial energy use in the Georgia Power regions that are mapped to the ten weather stations.

The regression model relates the total monthly industrial energy sales per calendar day to two monthly binary variables, six monthly weather interaction terms divided by the number of calendar days and total Georgia manufacturing employment.

Georgia Power is aware of a large industrial customer that will be coming on-line within the short-term forecast horizon. Since this new customer’s expected energy usage is not in the historical data, the model cannot predict its future usage. As a result, an external adjustment was made to the forecast beginning in March 2019 to account for the additional usage of this customer.

The forecast of industrial energy sales per calendar day is multiplied by the forecasted number of calendar days to get forecasted industrial calendar energy sales. A summary of the parameter estimates and statistics for the short-term industrial energy model is presented in Table 4.1.3-1.

Table 4.1.3-1: Industrial Energy Sales per Calendar Day

Short-Term Model

Budget 2019

Where:

## 4.1.4	MARTA Sales

MARTA energy sales, both short-term and long-term, are forecast with a linear regression model that relates sales to six monthly weather interaction terms. Table 4.1.4-1 presents the specific parameter estimates and statistics of the MARTA energy model.

Table 4.1.4-1: MARTA Energy Sales

Short-Term Model

Budget 2019

Where:

## 4.1.5	Governmental Lighting Sales

New outdoor lighting technologies being adopted throughout the Georgia Power system have made it necessary to use internal estimates for the short-term forecast rather than a single regression model relating governmental lighting energy sales to the growth in governmental lighting customers, as has been done in the past. In Budget 2019, it is assumed that by 2021, governmental lighting energy usage will be reduced by approximately 33% from 2014 levels. No explicit econometric model was used in the Budget 2019 forecast of governmental lighting.

# 4.2	CUSTOMER FORECASTS

Budget 2019 customer forecasts for the residential, commercial, industrial, and governmental lighting classes were developed using linear regression models.

## 4.2.1	Residential Customers

The Georgia Power residential customer forecast was developed using a regression model that relates the annual change in customers to the annual change in the number of Georgia households, a lag of the dependent variable and an intercept term. The annual forecasts are then spread across the months using an estimate of the monthly historical pattern. Table 4.2.1-1 presents the specific parameter estimates and statistics for the residential customer model.

Table 4.2.1-1: Residential Customers

Customer Model

Budget 2019

Where:

## 4.2.2	Commercial Customers

The Georgia Power commercial customer forecast uses the annual change in customers as the dependent variable in a regression model that includes the annual change in Georgia nonmanufacturing employment as the independent variable. The annual forecasts are then spread across the months using an estimate of the monthly historical pattern. Table 4.2.2-1 presents the specific parameter estimates and statistics for the commercial customer model.

Table 4.2.2-1: Commercial Customers

Customer Model

Budget 2019

Where:

## 4.2.3	Industrial Customers

The Georgia Power industrial customer forecast was developed using a regression model, with a one-month lag of customers as the explanatory variable. Table 4.2.3-1 presents the specific parameter estimates and statistics for the industrial customer model.

Table 4.2.3-1: Industrial Customers

Customer Model

Budget 2019

Where:

## 4.2.4	Governmental Lighting Customers

The Georgia Power governmental lighting monthly customer forecast was developed using a regression model that includes a one-month lag of customers and the total housing stock in Georgia as explanatory variables. A variable to capture changes in the billing system’s identification of Savannah Electric and Power Company’s governmental lighting customers in 2008 is also included. The specific parameter estimates and statistics for the governmental lighting customer model are presented in Table 4.2.4-1.

Table 4.2.4-1: Governmental Lighting Customers

Customer Model

Budget 2019

Where:

# 4.3	CALENDAR MONTH FORECAST

Georgia Power budgeting and accounting records are based on calendar month sales. However, meters are not read according to calendar months. Instead, they are read according to the billing schedule for each class. Separate calendar month sales forecasts, derived from billing month forecasts, are prepared for Georgia Power’s residential and commercial classes. Calendar month sales forecasts for the other retail customers are assumed to be equal to their billing month forecasts because their billing periods nearly coincide with actual calendar months.

For the short-term forecast (2019-2021), billing month energy sales are converted into calendar month energy sales by forecasting unbilled energy sales and applying the following equation:

Calendar Month Sales = Billed Sales (current month) + Net Unbilled Sales

where:

Net Unbilled Sales = Unbilled Sales (Current Month) – Unbilled Sales (Previous Month)

The unbilled sales forecast is estimated using the same forecast models for residential and commercial as presented in sections 4.1.1 and 4.1.2. All of the inputs are the same, with the exception of the monthly weather interaction terms in the forecast period. In the unbilled model, these monthly weather interaction terms are based on the number of unbilled days, rather than the number of billed days, to account for the impact of variations in the length of the unbilled period. As with the number of billing days, the number of unbilled days is based on the Company’s annual meter reading schedule.

The forecast of residential use per customer per unbilled day and commercial energy use per customer per unbilled day are multiplied by the forecasted number of customers and unbilled days to get the respective forecasted unbilled energy sales for each class to be used in the equation above to calculate calendar month sales.

# 5.0	LONG-TERM MODEL DESIGN

The long-term forecasts for the residential, commercial and industrial customer classes were produced with end-use models. The end-use approach allows for a detailed examination of the end uses that contribute to the energy usage of a customer class. For example, specific assumptions about appliance ownership, appliance efficiency improvements, and consumer choice characteristics are factored into the residential forecast.

For Budget 2019 the long-term forecasts used LoadMAP 4.0, developed by Applied Energy Group (“AEG”) (formerly EnerNOC Utility Solutions Consulting), for the residential, commercial and industrial sales classes. LoadMAP has been in use since the Budget 2015 forecast. For Budget 2019 the LoadMAP model used for the commercial class was updated with additional end uses.

The long-term twenty-year forecast for MARTA was produced using an econometric regression model.    The governmental lighting forecast was produced using judgment and internal expectations based on the plan to switch existing lights to more efficient LED lights. Detailed model specifications can be found in the short-term documentation, Section 4, while forecasted values can be found in Section 2.

Table 5.0-1 Model Type List

| Class | Short-Term | Long-Term |
| --- | --- | --- |
| Residential | Econometric | End Use |
| Commercial | Econometric | End Use |
| Industrial | Econometric | End Use |
| MARTA | Econometric | Econometric |
| Governmental Lighting | Judgment | Judgment |

# 5.1	LONG-TERM ENERGY SALES

## 5.1.1	Residential Long-Term Energy Sales

LoadMAP forecasts end-use or appliance-specific residential energy consumption using a variety of demographic, housing, economic, energy, and weather information. It also models energy consumption from three main components: housing stock served (customers), share of the housing stock served by appliance end-use by fuel type, and unit energy consumption of the appliance end-use by fuel type.  Appliance end-use by fuel type refers to an end-use class of appliances that consume a particular fuel (electricity or natural gas) such as electric clothes dryers or gas stovetops.

LoadMAP simulates household energy decisions with respect to three basic types of choices — appliance ownership by fuel type, appliance efficiency, and appliance utilization (energy usage). For a given end use and fuel type, consumption is the product of the number of households, the percentage of the segment (households) with the end use, and the average unit energy consumption. The equation that uses these three main components to determine consumption, or sales, is called the central energy equation.

Where:

Energy	= 	annual energy use in a particular segment (e.g., residential single-family homes)

t      	= 	technology

e        	= 	end use

N        	= 	market size (e.g., number of single-family homes)

Sharet  	= 	share of market with the end use (e.g., single-family homes with the end use). This is

also referred to as the end-use saturation.

UECt  	= 	unit energy consumption in unit of market size (e.g., single-family homes) with the end

use

Ue      	= 	utilization index (equals 1.0 in base year)

The elements of the central energy equation in the base year are contained in an energy market profile.

The appliance ownership shares used in the LoadMAP model are calibrated to Georgia Power’s service territory and initialized to a base year of 2007.

The residential energy model predicts energy consumption for 22 major household end-uses:

| 1. Cooling | 12. Dishwasher |
| --- | --- |
| 2. Heating | 13. Clothes Washer |
| 3. Water Heating | 14. Clothes Dryer |
| 4. Interior Lighting | 15. Personal Computer |
| 5. Exterior Lighting | 16. Monitor |
| 6. Refrigerator | 17. DVR |
| 7. Second Refrigerator | 18. Television |
| 8. Freezer | 19. Ceiling Fan |
| 9. Oven | 20. Well Pump |
| 10. Cooktop | 21. Pool Pump |
| 11. Microwave | 22. Other (Remaining energy-consuming activity) |

Additionally, the model is segmented by housing type (single-family, multi-family, and manufactured housing), and its logic distinguishes between new and existing housing behavior. Each customer group is described by the combinations of these attributes. As an example, new single-family housing demonstrates characteristic choice behaviors in the selection of the alternative appliances and fuel technologies available to the market. This choice behavior or demonstrated preference for a particular fuel choice technology in a given end-use category is measured by residential surveys.

For each of the end uses, LoadMAP forecasts equipment purchases, efficiency, and utilization choices.  Within the simulation, the probability of installing a given technology in a dwelling is dependent upon the operating and performance characteristics of the competing alternatives, as well as household and dwelling features. Certain end-uses are highly interdependent, such as heating and water heating, which typically use the same fuel type.

Average annual end-use consumption levels (often referred to as unit energy consumption or UEC) are from various national and regional sources including:  Energy Data Sourcebook for the U.S. Residential Sector, Lawrence Berkeley National Laboratory, the 2005 Residential Energy Consumption Survey (“RECS”) conducted by the U.S. Department of Energy (“DOE”), the 2008 Appliance Efficiency Rulemaking report by the California Energy Commission, the DOE’s 2009 Buildings Energy Data Book, and energy simulation tools. Final UECs are then calibrated to the base year. Average appliance size and efficiency by appliance vintage are from survey results and national shipment data compiled by Itron.

Appliance operating efficiency and utilization rates are simulated as interdependent decisions. Efficiency choice is dependent on operating cost at the planned utilization rate, while actual utilization depends on operating cost given the appliance efficiency. The sensitivity of efficiency and utilization decisions to costs, climate, household and dwelling size and income have been estimated from historical survey data.

Other exogenous variables include fuel prices (electricity and natural gas), average household income, household size, weather (measured in degree hours), natural gas availability, technology availability, and efficiency standards by end use. Saturation and penetration rates are from Georgia Power residential saturation surveys. Surveys, field observations, and other information are used to estimate gas availability.

The sources of inputs are varied. Historical natural gas prices are from the EIA State Energy Data System.

Forecasts of residential natural gas prices are developed by SCS Forecasting using the commodity price forecasts from SCS Fuel Services and SCS Resource Planning. Historical electricity prices are from Georgia Power’s records while projected electricity prices are based on the retail price from the 10-year financial model and extrapolated based on historical trends for the remaining years. LoadMAP uses a normal weather condition of cooling and heating degree hours in deriving the forecast. This weather condition is a thirty-eight-year average of cooling and heating degree hours from 1980 through 2017. Efficiency data are from the national default database developed by Itron, SCS Market Intelligence, EIA’s Technology Forecast Update and from several sources by AEG, including RECS data, and other national and regional data. LoadMAP data leverages AEG’s expertise providing the latest intelligence in efficiencies and technologies. Economic data are purchased from IHS Markit on an annual basis. Customers are separately forecasted at an aggregate level using the model presented in Section 4.2.1. This forecast is then disaggregated to single family, multi-family, and manufactured home using housing type trends from the IHS Markit housing stock forecast and calibrated to 2007 Georgia Power Residential Saturation Survey splits.

The residential long-term forecast is modeled on a billing basis. Budget 2019 is presented as a calendar-based forecast. The long-term billing forecast is integrated with the short-term billing forecast and then both are converted to a calendar basis. Attachment 5.1.1-1 shows the unadjusted long-term billing forecast, as well as the adjusted billed and calendar forecasts, which are adjusted for reductions in sales resulting from DSM and Behind-the-Meter (“BTM”) solar, and for the additional sales resulting from electric vehicles.

Several attachments follow which comprise the Budget 2019 Residential Long-term Forecast.

Attachment 5.1.1-1:  Total Residential Energy Sales (GWh)

Residential Long-Term Forecast

Budget 2019

- *In the following tables, values are based on the long-term billed end-use model. No external adjustments were made.

Attachment 5.1.1-2: GWh/BBtu Sales by End Use

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-2: GWh/BBtu Sales by End Use (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-2: GWh/BBtu Sales by End Use (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-3: GWh/BBtu Sales by End Use – Single Family

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-3: GWh/BBtu Sales by End Use – Single Family (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-3: GWh/BBtu Sales by End Use – Single Family (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-4: GWh/BBtu Sales by End Use – Multi Family

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-4: GWh/BBtu Sales by End Use – Multi Family (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-4: GWh/BBtu Sales by End Use – Multi Family (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-5: GWh/BBtu Sales by End Use – Manufactured Home

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-5: GWh/BBtu Sales by End Use – Manufactured Home (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-5: GWh/BBtu Sales by End Use – Manufactured Home (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-6: GWh/BBtu Sales by Fuel

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-7: Average UEC (kWh/BBtu)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-7: Average UEC (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-7: Average UEC (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-8: Average Share of Market

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-8: Average Share of Market (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

Attachment 5.1.1-8: Average Share of Market (cont’d)

Residential Long-Term Forecast (LoadMAP-R) Unadjusted

Budget 2019

## 5.1.2	Commercial Long-Term Energy Sales

The long-term forecast of electric energy sales for Georgia Power’s commercial class was produced using LoadMAP. The Commercial LoadMAP model uses floor space by building type and fuel price forecasts to produce a commercial energy sales forecast by different technologies for each end use and building type.

The commercial sector is segmented into thirteen building types, ten end uses, and two fuel types. The thirteen building types are:

| 1. Office | 8. Healthcare |
| --- | --- |
| 2. Restaurant | 9. Lodging |
| 3. Retail | 10. Miscellaneous Buildings |
| 4. Grocery | 11. Religion |
| 5. Warehouse | 12. Public |
| 6. Education | 13. Data Center |
| 7. Amusement |  |

The ten end uses modeled for each of the building types listed above are:

| 1. Primary Heat (Space Heating) | 6. Outdoor Lighting |
| --- | --- |
| 2. Cooling (Air Conditioning) | 7. Indoor Lighting |
| 3. Water Heating | 8. Office Equipment |
| 4. Cooking | 9. Ventilation |
| 5. Refrigeration | 10. Miscellaneous |

The two fuel types used for the commercial market are electricity and natural gas.

The Commercial LoadMAP model forecasts the energy consumption for each technology, in each end use in each market segment in the commercial class. Energy consumption is determined by building type, end use, and fuel type. Energy use patterns and end-use composition vary considerably between building types; therefore, building type is the primary differentiation of the commercial market. This segmentation is done to group together market segments with relatively common characteristics while separately identifying segments with distinctly different characteristics.

The differences in building type reflect differences in operating characteristics as well as differences in the composition and choice of energy consumption activities and energy consuming equipment and technology. These energy-consuming activities that deliver service to the occupants by various devices and equipment are called end uses. As identified above, examples of end uses include lighting, cooking, and heating and air conditioning. The composition and intensity of energy usage and the energy consuming activities within each building type vary according to the consumer activity and behavior resulting from the business purpose associated with the building. For example, a grocery store contains relatively more refrigeration and lighting while a restaurant has more energy consuming devices related to cooking. Therefore, the relative concentration of the different end uses varies by building type. Also, the energy consumption characteristics of each end use will vary by building type because the selection of equipment to deliver that end-use service is made as appropriate to the function of the activities inside the building.

Consumers have the opportunity to select alternative energy consuming equipment (devices and appliances) that deliver the same service (for example, water heating or cooking) but vary by fuel. Since capital and operating costs vary for similar devices of different fuel types, and consumer preference varies by fuel, fuel costs also have an impact in the selection of end-use equipment.

The Commercial LoadMAP model uses information about the operating characteristics, ownership decisions, and behavioral characteristics to determine energy consumption patterns of commercial customers. Factors influencing the energy consumption within a building include the operating characteristics of the building, preferences in the construction of the building, consumer behavior of the occupants of the building, and operational design and influences in the building such as energy management systems and controls.

It also models energy consumption using four main components:  building floor space, share of the floor space served by appliance end use and fuel type, energy consumption per square foot of floor space, and an index reflecting changes in energy usage of a fuel type. Appliance end-use by fuel type refers to an end-use class of appliances that consume a particular fuel (electricity or natural gas) such as electric cooking or gas room heating. The equation that uses these four main components to determine energy consumption, or sales, is called the central energy equation for the commercial class:

Energy = 	annual energy use in a particular segment (e.g., commercial restaurant)

t           = 	technology

e          = 	end use

N         = 	market size (e.g., amount of restaurant square footage)

Sharet  = 	share of market with the end use (e.g., restaurant floor space with the end use). This is also referred to as the end-use saturation.

UECt   = 	unit energy consumption in unit of market size (e.g., restaurant square footage) with the end use

Ue        = 	utilization index (equals 1.0 in base year)

The elements of the central energy equation in the base year are contained in an energy market profile.

The Commercial LoadMAP model uses economic, fuel price, market data, operating characteristics, and weather information to produce the energy forecast for the commercial class. The economic variable used is floor space by building type. The square footage forecast is done in-house by SCS and utilizes historical and short-term forecast square footage data from McGraw-Hill Construction and economic data from IHS Markit. It is the primary driver of energy consumption in the commercial class. Floor space is measured in square footage of building space.

There are two types of share definitions. The whole building concept measures the share of floor space in buildings with an end use regardless of the portion of each building that is served by the end use (for example, refrigeration in an office building). The second share concept refers to conditioned floor space and accounts for the fraction of each building that is conditioned by the end use (for example, air conditioning). The Energy Usage Index (“EUI”) is the average annual energy per square foot of floor space that is served by a particular end use and fuel type. The usage index (“UI”) is an index measured relative to the base year that indicates the relative amount of energy service (e.g. lighting) delivered for a specified end use and fuel type within a particular building type. The UI is determined by consumer (occupant) behavior, equipment controls, weather, and other operating characteristics. Consumer behavior and choice decisions such as changes in thermostat settings or operating hours can result in changes in energy usage.

The information for each technology’s saturation, energy use intensity, intensity per square foot, and sales for each end use comprise the market profile of energy use for a particular building type/fuel type combination. Market profiles of energy use for a building type are developed for each fuel type. End-use energy consumption by technology, or sales, is the product of intensity per square foot and floor space. The product of the saturation and EUI is called the building intensity or intensity per square foot. Market profiles show this decomposition of energy use. The information for the market profiles in the current Georgia Power commercial model is derived from the 2004 Georgia Power Commercial Saturation Survey, national and regional data, and additional substitutes as developed by AEG.

The numerical values of the components in this definition change over time. These changes reflect the economic decisions made in the commercial marketplace concerning the choice of building construction and building materials, installation of energy consuming end-use equipment, and usage patterns resulting from the consumer behavior of the occupants in the finished building. End-use modeling is concerned with capturing the effects of the primary influences on these decisions and projects the trend of each of these components throughout the forecast horizon.

Fuel share and EUIs reflect the choices among building and energy technology of investment decisions made by building owners, architects, and contractors at the time of new building construction or replacement of equipment. The choice of whether to include end-use equipment, along with which fuel powers the equipment or technology, affects fuel shares, end-use penetration, and energy consumption. Selection of building design options, energy-using equipment choice, building characteristics, and usage patterns determine the EUI for each fuel. Owner preferences, consumer behavior of the occupants of a building, and decisions made about the frequency and intensity of energy equipment use affect usage levels.

The Commercial LoadMAP model explicitly identifies some factors influencing the usage, such as fuel prices, weather data, operating hours, occupancy rates, and other information. Fuel prices and weather data are specific to the Georgia Power territory. Where Georgia Power data were not available, national and regional data were substituted.

Commercial LoadMAP models the economic investment decision process of a building owner. Investment decisions in the commercial market have a major role in the energy consumption within a building. The decisions regarding construction or equipment replacement for a building represent a trade-off between capital and operating costs of equipment. This trade-off is then reduced to decisions related to the fuel and equipment efficiency decisions of operating equipment and capital costs. LoadMAP uses a probabilistic method to model the decision process regarding which equipment system will be installed in a building. This method depends on factors such as the capital and operating costs of alternative options and building characteristics.

The historical energies for the commercial class are derived from Georgia Power specific data, distinguishing each segment using North American Industry Classification System (“NAICS”) codes.

Customers have the opportunity to purchase appliances and equipment that use a variety of fuels. The price of the fuel affects the usage patterns and purchasing habits of the decision maker. Electricity and natural gas prices are forecasted for the commercial sector and used in the Commercial LoadMAP model.

The model calibrates to the historical energy and segment information of the commercial class for the base year of 2007. Additionally, the long-term forecast produced is calibrated to anticipate changes in use-per-square foot in the commercial class.

The commercial long-term forecast is modeled on a billing basis. Budget 2019 is presented as a calendar-based forecast. The long-term billing forecast is integrated with the short-term billing forecast and then both are converted to a calendar basis. Attachment 5.1.2-1 shows the unadjusted long-term billing forecast, as well as the adjusted billing and calendar forecasts, which are adjusted for reductions in sales resulting from DSM, cogeneration and solar, and for the additional sales resulting from electric vehicles.

Several attachments follow which comprise the Budget 2019 Commercial Long-term Forecast.

Attachment 5.1.2-1: Total Commercial Energy Sales (GWh)

Commercial Long-Term Forecast

Budget 2019

*In the following tables, values are based on the long-term billed end-use model. No external adjustments were made.

Attachment 5.1.2-2: Sales by Building Type and Percentage of Class (GWh and Percentages)

Commercial Long-Term Forecast (LoadMAP-C) Unadjusted

Budget 2019

Attachment 5.1.2-3: Total Sales by End Use and Percentage (GWh and Percentages)

Commercial Long-Term Forecast (LoadMAP-C) Unadjusted

Budget 2019

Attachment 5.1.2-4: Average Share of Market

Commercial Long-Term Forecast (LoadMAP-C) Unadjusted

Budget 2019

Attachment 5.1.2-5: Cogeneration

Commercial Long-Term Forecast

Budget 2019

## 5.1.3	Industrial Long-Term Energy Sales

The long-term forecast of electric energy sales for Georgia Power’s industrial class was produced using LoadMAP. The Industrial LoadMAP model is used because of its capability to forecast at the end-use level and the increased effectiveness of resource utilization from using the LoadMAP family of long-term forecasting models. The Industrial LoadMAP model maintains consistency with the conceptual framework of the residential and commercial long-term models, while providing information appropriate to the industrial class.

The industrial sector is segmented into fourteen categories, based on the NAICS and nine end uses. The fourteen categories are:

| 1. NAICS 311 – SIC 20 Food | 8. NAICS 327 – SIC 32 Stone, Clay & Glass |
| --- | --- |
| 2. NAICS 313 & 314 – SIC 22 Textiles | 9. NAICS 331 – SIC 33 Primary Metal |
| 3. NAICS 321 – SIC 24 Lumber & Wood | 10. NAICS 332 – SIC 34 Fabricated Metal |
| 4. NAICS 322 – SIC 26 Paper | 11. NAICS 335 – SIC 36 Electronic Equipment |
| 5. NAICS 325 – SIC 28 Chemicals | 12. NAICS 336 – SIC 37 Transportation Equipment |
| 6. NAICS 324 – SIC 29 Petroleum | 13. Other Industrial |
| 7. NAICS 326 – SIC 30 Rubber & Plastic | 14. Unspecified |

Segment 13, “Other Industrial”, is comprised of multiple segments: Apparel (NAICS 315 – SIC 23), Printing (NAICS 323 – SIC 27), Industrial Machinery (NAICS 333 – SIC 35), Instruments (NAICS 334 – SIC 38), and Miscellaneous Manufacturing (NAICS 339 – SIC 39). Segment 14, the “Unspecified” category, comprises all remaining industrial activity.

The nine end uses modeled for each of the NAICS categories listed above are:

| 1. Cooling | 6. Process |
| --- | --- |
| 2. Heating | 7. Ventilation |
| 3. Indoor Lighting | 8. Miscellaneous |
| 4. Exterior Lighting | 9. Unspecified |
| 5. Motors |  |

In the industrial sector, energy requirements are largely related to the energy-to-output relationship, augmented by the share of alternative equipment choices and fuel options. In the industrial sector, economic output is measured by the amount of industrial production. Energy is required to manufacture the product. For example, a plant requires a certain amount of motive power (i.e. work from motors) or process heat to produce its product. A textile mill requires a certain amount of motive force (work) to weave a yard of fabric or make a carpet. A plant that melts steel requires a certain amount of energy to melt a ton of steel, which is determined by the process efficiency of converting fuel sources into useful work. These energy requirements are supplied by energy consuming equipment. The amount of energy consumed by the equipment to meet this need is the amount of end-use sales. By installing more efficient equipment, a plant operator can lower the amount of energy needed to meet the requirements of the process. The end-use machinery that supplies the energy requirements of the plant needs a fuel source itself. This could be one of several fuels:  electricity, natural gas, oil, or some other fuel. The plant is typically designed and operated to use a combination of equipment that uses different fuel sources and maximizes efficient use of resources while providing end-use service economically and reliably.

The long-term industrial model uses the elements of economic activity, industrial production operations, fuel choices, and equipment (or process) characteristics to model energy requirements arising from the supporting end uses of an industrial activity. The description of the economic activity and production operations of an industry come from industrial production, capacity utilization rates, and output capacity. Output capacity measures the level of output of plants when they are operating at full capacity. From this, a description of the energy required to support a given level of production can be made. Fuel prices are used to model the choice of end-use equipment employed to provide end-use service. These choices, although influenced by the preferences of the decision maker, are largely economic and are captured through analyzing life-cycle costs. The capital costs of equipment also affect the selection of equipment. In addition, the model uses information about the mix of equipment options, sizes, and ratings to determine end-use consumption characteristics.

Efficiencies, equipment size, and usage characteristics determine consumption. Equipment purchase decisions result in the stock and efficiency mix of new equipment. The first type of equipment replacement decision is made at the time of physical decay of a device and is modeled using assumptions about equipment lifetimes. The second type of equipment purchase decision results from plant construction or expansion and is modeled using changes in capacity. Share equations are used to compute market shares for the various efficiency options and are based on a logic function describing equipment purchases based on life cycle costs. Data from available surveys as well as national and regional information are used to describe these parameters for each industry.

The central energy equation for the Industrial LoadMAP model is:

Energy = 	annual energy use in a particular segment (e.g., Chemicals)

t           = 	technology

e          = 	end use

N         = 	market size (e.g., Industrial Production Index level for Chemicals)

Sharet  = 	share of market with the end use (e.g., chemical IP index level with the end use). This is also referred to as the end-use saturation

UECt   = 	unit energy consumption in unit of market size (e.g., Chemical IP Index) with the end use

Ue        = 	utilization index (equals 1.0 in base year)

The elements of the central energy equation in the base year are contained in an energy market profile.

The economic driver used to forecast long-term sales for each NAICS category in the industrial class is industrial production, as measured and tracked by the Federal Reserve. The use of industrial production allows for the explicit modeling of the input-output structure of the economy. Consequently, by using industrial production in the energy models, energy consumption is directly linked to a measure of industrial economic output. Section 8.2.4 addresses industrial activity.

The historical energies for the industrial class are derived from Georgia Power specific data, distinguishing each segment using NAICS codes. Electricity and natural gas prices are forecasted for the industrial class.  Other data, such as fuel shares, efficiencies, technologies, etc. were implemented from several sources by AEG, including the Manufacturing Energy Consumption Survey (“MECS”), and other national and regional data.

The Industrial LoadMAP model uses a base year of 2007 to establish initial calibration. The model also calibrates to historical energy through 2017. Additionally, it is calibrated to the energy of each economic category, according to NAICS, over the period of the short-term forecast, 2018 through 2021.

Cogeneration is a significant activity for certain industries. In the course of providing general manufacturing production processes, electricity is made available as a by-product. Georgia Power has assembled information to project the energy and demand changes from cogeneration for the near future. This cogeneration forecast is produced independently of the electricity sales forecast of the industrial class. See Section 8.4.2 for additional discussion on cogeneration. The amount of industrial cogeneration expected in the Budget 2019 is presented in Attachment 5.1.3-4.

The industrial long-term forecast is modeled on a billing basis. Budget 2019 is presented as a calendar-based forecast. The long-term billing forecast is integrated with the short-term billing forecast and then both are converted to a calendar basis. Attachment 5.1.3-1 shows the unadjusted long-term billing forecast, as well as the adjusted billing and calendar forecasts, which are adjusted for reductions in sales resulting from cogeneration and solar.

Several attachments follow which comprise the Budget 2019 Industrial Long-term Forecast.

Attachment 5.1.3-1: Total Industrial Energy Sales (GWh)

Industrial Long-Term Forecast

Budget 2019

*In the following tables, values are based on the long-term billed end-use model. No external adjustments were made.

Attachment 5.1.3-2: Industrial Sales by NAICS Category (GWh and Percentages)

Industrial Long-Term Forecast (LoadMAP-I) Unadjusted

Budget 2019

GWh Sales

Attachment 5.1.3-2: Industrial Sales by NAICS Category (GWh and Percentages) (cont’d)

Industrial Long-Term Forecast (LoadMAP-I) Unadjusted

Budget 2019

Percentage of Total Sales

Attachment 5.1.3-3: Industrial Sales by End Use (GWh and Percentages)

Industrial Long-Term Forecast (LoadMAP-I) Unadjusted

Budget 2019

Attachment 5.1.3-4: Cogeneration

Industrial Long-Term Forecast

Budget 2019

# 5.2	ANNUAL TO MONTHLY BASED ENERGY FORECASTS

The official Budget 2019 forecast is one of monthly calendar energies. However, the long-term end-use models are of an annual frequency and based on billing data which results in annual billing energy forecasts.

The long-term end-use annual forecasts were converted to monthly forecasts by allocating each annual forecasted value per its class monthly percentage (see Table 5.2-1). For all three classes, the monthly percentages from the last year of the short-term models were used as the monthly pattern in the long-term projections. The result is a 20-year monthly billed forecast. This billed forecast is converted to a calendar forecast using the monthly billing-to-calendar ratios in Table 5.2-2.

Table 5.2-1: Annual to Monthly Allocation Percentages

Table 5.2-2: Monthly Billing to Calendar Ratios

# 5.3	INTEGRATION OF SHORT- AND LONG-TERM ENERGY FORECASTS

The short-term and long-term energy forecasts were produced using different methods. Although both the long-term and short-term energy models used a 38-year average as the predicted weather, the impacts from using different forecast methodologies for the two periods increase the likelihood that integration techniques are needed to smooth out the forecast. The forecast should be smooth because the timing of economic, demographic, and weather events further into the future from historical results becomes less and less certain. Adjustments to ensure a smooth transition of the various elements of the forecast and between years of the forecast were made to the energy forecast during the development of the energy forecast. Specifically, the long-term models are calibrated to historical and short-term model results for integration purposes.

# 6.0	PEAK DEMAND FORECAST

Georgia Power used the PDM in Budget 2019 to predict peak demands. The PDM is a SAS product and was developed in a partnership of SAS and Corios Group for Southern Company in 2013-2014. The PDM was first used in Budget 2015.

PDM uses historical hourly load research data for each customer class to derive functions that describe the relationship of load to the corresponding weather. Day types and seasons are also defined for the calendar period(s). To predict demands, the following components are used: the PDM class load profiles, the relationships of load and weather developed in PDM from the historical load research data, the description of “typical” weather and the class monthly energy forecast. Both the coincident and non-coincident demands by class are produced with the PDM model.

The Georgia Power PDM uses load shapes primarily based on class to define its territory. Currently, there are the five primary classes: Residential, Commercial, Industrial, Government Lighting and MARTA; plus three additional categories: Electric Vehicles, Behind the Meter Solar, and one expected new large industrial customer. Most of the class input historical load shapes use load research historical data from 2015 & 2016. The Electric Vehicle, BTM Solar, and the new large industrial customer require several specific load shapes to properly capture their expected behavior. In Budget 2019, four load shapes were used for Electric Vehicles, three were used for BTM Solar, and two for the expected new large industrial customer.

The methodology determining “typical” weather uses 38 years of average daily temperatures which were rank ordered and then averaged to form a “temperature duration curve.” The most representative pattern of actual temperatures was chosen using a least sum of squares criteria. The temperature duration curve was then re-sorted to the chronological order of the representative pattern.

The final component in the development of the demand forecast is the input of the monthly calendar-based energies. The forecasted energies are based on a 38-year average weather assumption. Using the forecasted energies directly in the peak demand model maintains consistency between projected energy sales and the forecast demands.

PDM also expands the monthly sales data to supply level (sales plus losses) for generation required for the projected energy and demands. PDM uses expansion factors, in Attachment 6.0-5, to go from the customer level to the supply level energy and demands. Georgia Power uses the results as the supply forecast. The forecasting demand and energy expansion factors used in Budget 2019 were derived using the results from the Georgia Power 2015 loss study with updated 2016 peak and energy load data.

Estimates for RTP response, cogeneration, and DSM are made for the reduction in demand and incorporated as external adjustments to the peak demand forecast. Sections 8.4.1 through 8.4.5 contain more information on adjustments made to the energy and peak demand forecasts.

Attachment 6.0-1: Annual Coincident Demand with Adjustments (MW)

Calendar Based

Budget 2019

* Georgia Power is a summer peaking utility over the entire forecast horizon.

Attachment 6.0-2: Annual Unadjusted Non-Coincident Demand (MW)

Calendar Based

Budget 2019

Attachment 6.0-3: Monthly Coincident Demand with Adjustments (MW)

Calendar Based

Budget 2019

Attachment 6.0-3: Monthly Coincident Demand with Adjustments (MW) (cont’d)

Calendar Based

Budget 2019

Attachment 6.0-4: Monthly Unadjusted Non-Coincident Demand (MW)

Calendar Based

Budget 2019

Attachment 6.0-4: Monthly Unadjusted Non-Coincident Demand (MW) (cont’d)

Calendar Based

Budget 2019

Attachment 6.0-5: Expansion Factors

Budget 2019

Attachment 6.0-6: Annual Supply with Adjustments (GWh)

Calendar Based

Budget 2019

Attachment 6.0-7 Monthly Supply by Class (GWh)

Calendar Based

Budget 2019

Attachment 6.0-7 Monthly Supply by Class (GWh) (cont’d)

Calendar Based

Budget 2019

Attachment 6.0-8: Coincident & Non-Coincident Historical Peak Demand by Class (MW)

Attachment 6.0-8: Coincident & Non-Coincident Historical Peak Demand by Class (MW) (cont’d)

# 7.0	SENSITIVITY AND SCENARIO DEVELOPMENT OVERVIEW

Budget 2019 is a base case forecast using the economic forecast from IHS Markit, a 38-year average normal weather definition, and an electricity price forecast based on results of financial planning models and information from the Department of Energy. The economic forecast for the base case represents a description of the economic environment that is most likely to take place absent climate legislation or new environmental rulemaking. There are many possible events or actions that could occur, however, to change the characteristics of the economy or business environment of Georgia Power and, consequently, forecasted energy sales. Similarly, there is a possibility of deviation from the price assumptions and current regulatory environment. In order to evaluate the impact of changes to some of these key assumptions and variables, sensitivities around the key forecast variables and scenarios around potential carbon and fuel price outcomes were developed.

# 7.1	SENSITIVITIES

In the current application, a sensitivity is defined as a specified collection of univariate changes occurring coincidentally. Five sensitivities were developed for Budget 2019 relative to the "Base Case" forecast. The sensitivities presented below include: 1) high economic growth; 2) low economic growth; 3) no load growth; 4) load growth with no DSM; and 5) load growth with aggressive DSM. The high and low economic sensitivities are based on the Optimistic and Pessimistic scenarios developed by IHS Markit. These sensitivities were performed to identify impacts to the energy and peak demand forecasts pursuant to the specified changes in economic variables utilized in the energy forecast.

The variables and results of the sensitivities are identified in the following attachments.

Attachment 7.1-1: Annual Energy Sensitivity Summary

Attachment 7.1-2: Annual Peak Demand Sensitivity Summary

# 7.2	SCENARIOS

Nine scenarios were developed for Budget 2019 to evaluate the effects of different potential combinations of CO2 and fuel forecasts. The fuel scenarios are: 1) Low; 2) Medium (the basis for the “Base Case” energy forecast); and 3) High. Potential carbon pressure was modeled as a tax on the economy at varying prices resulting in differing economic conditions. The levels of the carbon cost used are: 1) $0 CO2 (the basis for the “Base Case” energy forecast); 2) $10 CO2; and 3) $20 CO2. Charles River Associates (“CRA”), a leading economic consulting firm, was engaged to develop economic models that could be used to investigate the effects of various carbon costs on the economy.

Each of the nine cases was created by comparing the “Base Case” residential, commercial, and industrial outcomes with respect to electricity demand with the same variables in the scenario case. Multipliers derived from this comparison were used to adjust residential, commercial, and industrial energy use and demand forecasts at the Southern Company level using LoadMAP and PDM. Southern Company results were then allocated across the four retail operating companies, Georgia Power, Alabama Power, Mississippi Power, and Gulf Power.

These scenarios were done to identify impacts to the energy and peak demand forecasts pursuant to the matrix of fuel forecasts and carbon costs.

The formulaic description for the process for each forecast year is:

• CRA change case electricity demand ÷ CRA base case electricity demand = Scenario variable multiplier

• Unadjusted Budget 2019 class-level energy in end-use model x Scenario variable multiplier = Adjusted class-level energy used for scenario energy forecast

Once all years are calculated for all classes, they are combined for the Southern Company scenario energy forecast.

The CRA model outputs include estimated impacts on retail energy by customer class and the CRA energy changes can be applied directly to internal energy projections. Estimating the effects of natural gas/carbon cost combinations on peak demand is more difficult than on energy use, because changes in energy by class will typically result in a change in the class composition of the total system shape. The shares of residential, commercial, and industrial energy use in the total system will generally change. Each of these customer classes exhibits different hourly patterns, and these differences in load shapes mean that the change in total peak demand will not equal the change in total energy. Because of time and resource constraints, it was impractical to re-run each of the operating companies’ PDM models for all scenarios. As a result, a simplified method of estimating a new total system peak demand was developed.

If the diversity factor for each class remains constant across the various scenarios, a new total system peak demand can be estimated by calculating a new non-coincident peak (“NCP”) and a new coincident peak (“CP”) for each class.

The change in total peak demand will differ from the change in total energy if there is a change in the class composition. But, for a single class, the change in NCP is the same as the change in energy if a constant load factor is a function of the CP of that class and the CP is a function of the NCP and the diversity factor for that class.

The key assumptions for this result are: 1) for each class, load factors are not affected by changes in energy, and 2) diversity factors for each class are not affected by changes in the total system’s class composition for energy. Diversity factors remain the same even if energy shares by class change.

The next page contains an illustration of the simplified method of estimating the new total system peak for each scenario.

Example:  Assume a given system with the following:

|  | Residential | Commercial | Industrial | System |
| --- | --- | --- | --- | --- |
| Energy (GWH) | 10,000 | 10,000 | 10,000 | 30,000 |
| Load factor | 40.0 % | 60.0 % | 80.0 % |  |
| Diversity | 90.0 % | 95.0 % | 98.0 % |  |

The NCPs and CPs are as follows:

|  | Residential | Commercial | Industrial | System |
| --- | --- | --- | --- | --- |
| NCP (MW) | 2,854 | 1,903 | 1,427 |  |
| CP (MW) | 2,568 | 1,807 | 1,398 | 5,774 (Load factor = 59.3%) |

New NCPs and CPs result from the indicated changes in energy use:

|  | Residential | Commercial | Industrial | System |
| --- | --- | --- | --- | --- |
| Change (%) | -5.0 % | -10.0 % | -15.0 % |  |
| Energy (GWH) | 9,500 | 9,000 | 8,500 | 27,000 (-10% change) |
| NCP (MW) | 2,711 | 1,712 | 1,213 |  |
| CP (MW) | 2,440 | 1,627 | 1,189 | 5,255 (-9.0 % change) |

Attachments 7.2-1 and 7.2-2 reflect Georgia Power’s portion of the Southern Company System Scenario Summary.

Attachment 7.2-1:  Annual Energy Scenario Summary

Budget 2019

Attachment 7.2-2:  Annual Peak Demand Scenario Summary

Budget 2019

# 8.0	FORECAST ASSUMPTIONS OVERVIEW

Budget 2019 assumptions were jointly developed by Georgia Power’s Market Planning and Financial Planning and Analysis departments, along with the SCS Forecasting, Marketing, Fuel Services, and System Planning departments. Major assumptions include the economic outlook for the U.S. and Georgia, as well as the long-term outlook for fuel prices and market profiles for class end uses.

IHS Markit provided the economic forecasts for Budget 2019. The economic forecast was developed in the Spring of 2018. IHS Markit provides comprehensive and timely research of all the components that drive the economy. IHS Markit’s research examines the implications of macro and regional growth, the prospects of states and metropolitan areas and covers specialized topics such as housing, consumers and other trends highly relevant to business planning. Clients from a broad range of industries depend on IHS Markit’s research and forecasts to better manage the risks and opportunities stemming from a rapidly changing economy, as well as for strategic planning, demand assessment, consumer lending and general investment research.

IHS Markit’s national model is a large-scale structural econometric model that is grounded in mainstream economic theory and estimated using modern econometric techniques. The model uses an income-expenditure structure in which short-term fluctuations in GDP are caused primarily by changes in aggregate demand. A distinguishing characteristic is that the steady-state properties of key equations are derived from neoclassical theory, imparting to the model a well-defined growth path in the long run. This emphasis on theory leads to an internally consistent structure that renders the model well-suited for short-run forecasting, long-term policy analysis and the development of internally consistent macroeconomic scenarios. With respect to regional modeling, each area (e.g., state or metro area) is modeled individually and then linked to a national system. Instead of forecasting regional growth as simple proportions of U.S. totals, the models focus on each area’s internal growth dynamics and differential business cycle response.

As a part of the forecasting process, prices for electricity and natural gas are reviewed and analyzed. Electricity price forecasts were based on results of internal financial planning models. Projections of natural gas prices were developed by SCS Forecasting using the commodity forecasts from the SCS Fuel Services and SCS Resource Planning groups. Georgia Power’s long-term forecasting models incorporate these retail fuel prices when modeling the fuel choices and efficiency decisions of consumers and decision-makers.

Other factors, such as the impacts of appliance efficiency standards, were also considered in the long-term forecast. The end-use models for the long-term energy forecast incorporate minimum standards for new appliances as well as replacement options.

The short-term and long-term energy models use a 38-year average of CDH and HDH to define “normal” weather.

# 8.1	SHORT-TERM ENERGY MODEL ECONOMIC ASSUMPTIONS

The Budget 2019 short-term models use economic and demographic inputs from the Georgia forecast produced by IHS Markit. The short-term energy forecasts rely on the monthly interpolations derived from quarterly projections of demographic and economic variables, including the number of Georgia households, and Georgia manufacturing and non-manufacturing employment.

# 8.2	LONG-TERM ENERGY MODEL ECONOMIC ASSUMPTIONS

## 8.2.1	Overview

The Budget 2019 long-term energy models used economic and demographic inputs developed from the Georgia forecast produced by IHS Markit. The key variables used in the long-term energy models are discussed below.

## 8.2.2	Income per Household

The long-term residential energy forecast is produced using real income per household by housing type as the key economic driver. It represents single-family income per household. For multifamily and manufactured homes, income per household is calculated as a percentage of single-family income based on 2005 RECS data. It is assumed that this economic metric adequately represents the household income for the residential customers of Georgia Power.

In the Budget 2019 long-term commercial forecast model, income per household is used as the primary economic driver in the Restaurant and Retail sectors, which accounts for the impact of income on these commercial sectors.

## 8.2.3	Floor Space (Commercial Activity)

The Commercial LoadMAP model uses a floor space (square footage) forecast by building type to produce the long-term forecast of commercial energy sales.

## 8.2.4	Industrial Production (Industrial Activity)

The Industrial LoadMAP model used in Budget 2019 uses annual industrial production by selected NAICS categories as a driver to forecast long-term energy sales. This driver describes the output of each NAICS category in the industrial class. Since the energy requirements of each group are closely related to the quantity of output produced, this measure is a good choice for a driver of the industrial forecast. The base year consumption (GWh) provides a scale for each segment, which is then modified by the Industrial Production Index over time.

## 8.2.5	Demographics

Demographic trends in Georgia are an integral part of the Georgia economy and play an important role in energy consumption trends. Consequently, several demographic variables are key indicators for the various energy and customer models used in Budget 2019 such as the number of households and housing stock.

### 8.2.5.1 Persons per Household

The Residential LoadMAP model uses annual projections of the number of persons per household. It represents single-family persons per household. Persons per household for multi-family and manufactured homes are calculated as a percentage of single-family persons per household based on 2005 RECS data. This economic metric represents the characteristics of household size for the residential customers of Georgia Power.

### 8.2.5.2 Housing Type Distribution

The long-term residential energy model is configured to forecast energy sales by housing type. The three housing types modeled are single-family, multi-family, and manufactured homes. Historical and forecast values for the Georgia Power residential customer housing stock distributions by housing type were based on the 2007 Georgia Power Residential Saturation Survey and IHS Markit’s housing stock forecast.

### 8.2.5.3 Commercial Employment Rate (Commercial Activity)

The long-term commercial energy model uses the employment rate as a driver of the long-term energy forecast. The employment rate is derived by subtracting the Unemployment Rate from the total percentage (i.e. Employment Rate = 1 – Unemployment Rate). Unlike Floor Space, it is not calculated by building economic categories (building types). However, the long-term commercial model does account for the fact that the impact of the employment rate varies across the commercial sectors.

# 8.3	FUEL PRICE ASSUMPTIONS

## 8.3.1	Forecasts by Fuel Type: Electricity and Natural Gas

Forecasts of retail fuel prices were required for the energy models of three customer classes: residential, commercial, and industrial.

The long-term residential, commercial, and industrial energy models require forecasted fuel prices as an exogenous input. The electricity price forecasts are based on results of financial planning models produced by Georgia Power’s Financial Planning and Analysis group. These models produce electricity price forecasts assuming a specific company construction plan, the financial needs of Georgia Power, and economic conditions. The electricity price forecasts were developed in the spring of 2018. The natural gas prices are developed based on commodity price projections from SCS Fuel Services and SCS Resource Planning. The long-term forecasting models incorporate these retail fuel prices when modeling the fuel choices and efficiency decisions of consumers and decision-makers. Electricity and natural gas retail price projections for the residential, commercial, and industrial classes are presented in Attachment 8.6-1.

## 8.3.2	Substitution of Electricity with Competing Fuels

Fuel choice is an important factor to consider in the development of the forecast. Each customer responds, in various ways, to the price of electricity relative to the price of alternative fuels. The responsiveness of the customer, or decision-maker, depends on the customer’s preference and ability to change. These factors are recognized and are incorporated into the Georgia Power forecast.

The residential, commercial, and industrial long-term energy models all use the same modeling and conceptual framework. Using a logit model, the choice of new technology is determined by evaluating the life cycle of each technology for a given end-use. The life-cycle cost is a function of capital cost plus operating costs and any additional O&M costs discounted for the time value of money.

As the prices vary among the alternative fuels (e.g., electricity and gas), the operating cost will also vary and will affect the choices of end-use fuel types made by the consumer. Likewise, as the capital cost of the technologies of alternative fuel choices vary, the choices of technology will also vary. In this manner, the long-term commercial and industrial energy models capture the effects of competing fuels in end uses.

# 8.4	FORECAST ADJUSTMENTS

Demand side strategies and customer actions, existing Distribution Efficiency Program (“DEP”) impacts, and the effects of Time of Use (“TOU”) rates are captured directly in the forecast since they are in the historical data. The impacts of RTP response, cogeneration, new DSM programs, electric vehicles and BTM solar are captured in the forecast through adjustments outside of the forecast models.

## 8.4.1	Demand Side Strategies and Customer Actions

Budget 2019 incorporates adjustments for load responses by customers to changes in price, as well as for demand side programs being offered to customers that are anticipated to result in reductions in energy use and in peak demand. There are two types of impacts incorporated directly in the forecast: (1) the historical set of customer actions embedded in the historical data that are implicitly propagated throughout the forecast, and (2) adopted efficiency standards in the residential and commercial end-use models. Additional DSM adjustments are made to reflect new or ongoing Georgia Power programs that are not fully reflected in the historical data.

Georgia Power offers energy-efficiency programs to its residential and commercial customers to promote energy savings. These programs have existed for more than a decade and the benefits from the programs are embedded in Georgia Power’s historical load and energy sales. Therefore, the forecasts produced by using historical energy data and the embedded energy effects have an implicit effect propagated throughout the forecast. Other conservation measures undertaken by customers, whether explicitly induced by a marketing program or proactively undertaken by a customer for some other reason will become embedded in the historical energy data and will be implicitly propagated throughout the forecast.

An additional type of demand side impact considered in Budget 2019 is the adopted efficiency standards in the residential and commercial end-use models. The efficiency standards represent the continuing trend of increasing end-use efficiency driven in part by programs such as ENERGY STAR®.

## 8.4.2	Cogeneration

The effects of cogeneration are incorporated into Budget 2019. Available near-term information for known and expected cogeneration projects has been assembled. The long term commercial and industrial models were developed as the basis of electricity sales without assumptions for new cogeneration. Adjustments to Budget 2019 for the impacts of additional (new) cogeneration were made exogenously and were applied to the commercial and industrial sales forecasts beginning in 2018. Attachments 8.6-2 (cogeneration energy reduction at meter) and 8.6-3 (cogeneration energy reduction at generator) each provides a table of the projected impact of cogeneration on energy sales.

Information for known and expected increases in cogeneration was obtained by Georgia Power Sales and Service personnel from interviews with customers with either existing cogeneration or an announced project. A probability that each reported new cogeneration project will actually materialize was estimated for each. The probability-weighted MW announcements were then used to project the amount of additional cogeneration to incorporate into the forecast. The estimated MW cogeneration load was then converted into MWh energy generation using a common load factor. Attachments 8.6-4 (cogeneration demand reduction at meter) and 8.6-5 (cogeneration demand reduction at generator) each provides a table of the projected cogeneration impacts on demand. Since cogeneration reduces energy sales and peak demand, the amounts in these tables are subtracted from the energy and peak forecasts as an external adjustment.

## 8.4.3	Real Time Pricing and Time of Use

The RTP rate is designed to enable customers to take advantage of the variations in hourly system marginal costs. This gives the customer the opportunity to control their own cost in using electricity by changing their load shape in response to hourly price signals. Current estimates based on analyses of customer responsiveness to the RTP rate indicate a reduction to peak demand of REDACTED MW in 2019, increasing to a reduction of REDACTED MW in 2038. RTP response is captured in the forecast through an external adjustment. A model developed by Christensen Associates Energy Consulting is used to estimate the RTP price responsiveness and its impact on load.

TOU rates also give customers the incentive to control their costs by the differentiation of the prices of energy depending on time of day. Customers have been on TOU rates for a number of years and the impact of their usage patterns is being captured in the class load shapes that were used in development of the peak demand model.

## 8.4.4	Electric Vehicles

The impacts of expected growth in electric vehicles in Georgia have been incorporated into the Budget 2019 forecast. The impact of existing electric vehicles is already embedded in the historical data that is implicitly propagated throughout the forecast. Electric vehicles represent a positive adjustment to the forecast and represents expected growth in the number of electric vehicles in the state of Georgia. The forecast of electric vehicles in Georgia is based on a scenario by EPRI, which produces forecasts for states across the nation. Georgia Power selected the ‘medium” EPRI scenario for use in Budget 2019.

Electric vehicles are incorporated into the Budget 2019 peak demand forecast using load shapes for the residential and commercial classes.

## 8.4.5	Behind-The-Meter Solar

BTM solar is a solar energy system that produces power intended for on-site use in a home, office building, or other commercial or industrial facility. Thus, a BTM system allows an owner to use the energy produced from their system first, before using energy from the grid. Such systems directly reduce the energy consumed and the demand from the residential, commercial and industrial classes. An external adjustment is made to the energy forecast to account for the impact of BTM solar. BTM solar directly reduces the amount of energy that would be purchased from the grid.

BTM solar is incorporated into the Budget 2019 peak demand forecast using load shapes for the residential, commercial and industrial classes.

# 8.5	IMPACTS OF APPLIANCE EFFICIENCY STANDARDS

The efficiency of technologies and devices selected by decision-makers and consumers has a substantial impact on the energy requirements of the forecast. Market forces and conditions, economic valuation, and convenience have considerable impact on the choice of technologies. Various state and federal regulatory bodies and legislation, however, have also made provisions to ensure increases in technology efficiency. Notable instruments that serve to ensure the efficient use of electricity and thermal building integrity include the Georgia State Energy Code for Buildings, the National Energy Policy Act of 1992 (“NEPA”, also known as “EPACT”), the Energy Policy Act of 2005, the National Appliance Energy Conservation Act of 1987 (“NAECA”) and the Energy Independence and Security Act of 2007 (“EISA”).  In addition to these, new review and rule-making proceedings are routinely conducted at the state and federal levels.

These Acts contain a wide variety of provisions that will affect the demand of electricity, specifically that of HVAC and lighting technologies. In 2006, the minimum air conditioning efficiency standard was changed from SEER 10 to SEER 13. From 2012 to 2014, EISA established new lighting standards requiring new bulbs to be at least 25% more efficient than traditional incandescent bulbs. The types of bulbs fitting these standards include: halogen incandescents, compact fluorescents and LEDs. In 2012, the National Cable and Telecommunications Association (“NCTA”) signed the Voluntary Agreement for Ongoing Improvement to the Energy Efficiency of Set-Top Boxes, which agreed to start selling Energy Star Version 3.0 compliant set-top boxes. From 2013 to 2015, the Association of Home Appliance Manufacturers (“AHAM”) implemented new efficiency standards for various home appliances. In 2014, electric water heater efficiency improved from 90% to 95%. In 2015, the minimum SEER standard for the southern U.S. improved from SEER 13 to SEER 14. In 2016, the DOE finalized new standards for commercial rooftop air conditioning units, which is a two-step process going into effect in 2018 and 2023 raising the integrated energy efficiency ratio to higher levels based upon the size of the unit. Also, in 2017, the DOE approved new standards for Walk-In Coolers and Freezers that will take effect in 2020.

# 8.6	Budget 2019 Energy Prices and Forecast Adjustments

Attachment 8.6-1: Class Retail Energy Prices

Attachment 8.6-2: Cogeneration Energy Adjustments – Energy / Meter (MWh)

Energy Requirements at the Meter by Month

Budget 2019

Attachment 8.6-3: Cogeneration Energy Adjustments – Energy / Generator (MWh)

Energy Supply at the Generator by Month

Budget 2019

Attachment 8.6-4: Cogeneration Peak Demand Adjustments. – Territorial / Meter (MW)

Georgia Territorial Peak Demand at the Meter by Month

Budget 2019

Attachment 8.6-5: Cogeneration Peak Demand Adjustments – Territorial / Generator (MW)

Georgia Territorial Peak Demand at the Generator by Month

Budget 2019

# 8.7	RESOURCE SIDE DEMAND SIDE OPTIONS

Demand Side Options (“DSO”) that are used as capacity resources do not affect the Peak Demand forecast directly. DSOs that fall into this category include the Power Credit Direct Load Control (“DLC”) program, the Demand Plus Energy Credit (“DPEC”) interruptible tariff, Conservation Voltage Reduction (“CVR”), and Extreme RTP Response.

# 9.0	FINAL IRP REQUIREMENT OVERVIEW

This section is a supplement to the Budget 2019 documentation book (the standard documentation) for the Georgia Power Budget 2019 Load and Energy Forecast. It supplements the standard documentation to ensure the IRP rules are completely satisfied.

# 9.1	DESCRIPTION OF WEATHER NORMALIZATION

The purpose of weather normalizing energy and peak demand is to isolate the portion of variance between forecasted and actual values that arise from factors other than weather. Methods for accounting for variances from weather for energy are similar to those used for peak demand. Both are appropriate and justifiable.

Traditionally, energy forecasts are built using econometric models. That is, energy sales may be dependent on some economic and demographic variables and weather. A regression model will yield parameters that explain that relationship. To build an econometric forecast of energy sales requires a forecast for the economic and demographic variables and weather. For weather, the assumption is that future weather will be defined as the average of the prior 38 years, which is termed “normal weather.” Then, using normal weather with the regression models yields a prediction of energy sales under average weather conditions. To weather normalize after a period is over requires only the actual weather to be used with the model. The difference between the normal weather-predicted energy sales and the actual weather-predicted energy sales is the amount of variance due to weather.

The model developed for the peak demand forecast is the same model used for the peak demand weather normalization. The peak demand forecast model was built in PDM using “normal weather” and forecasted energies, which are based on “normal weather.” To weather normalize, actual weather is input into this same PDM model; it is analogous to an “actual weather” scenario. The difference between the normal weather-predicted peak demand and the actual weather-predicted peak demand is the amount of variance due to weather.

# 9.2	REVIEW OF ECONOMIC INPUT AND LOAD PROFILES

Georgia Power’s forecasting organization carefully reviews all forecast inputs from internal and external sources for reasonableness and accuracy and requires providers to document assumptions and justify results that represent significant deviations from previous trends or expectations. If the supporting documentation or explanation is inadequate, Georgia Power works with the information provider to reach a mutually agreed upon solution.

For Budget 2019, the Georgia Power forecasting group reviewed all economic forecast inputs provided by IHS Markit and was satisfied that the economic forecast inputs were reasonable and consistent with those of other economic forecasters at the time the energy and load forecasts were developed.

There are three areas of particular concern in previous IRPs that continue to be monitored – 1) the number of persons per household, 2) growth in average household income, and 3) the level of miscellaneous household energy consumption. Comparisons of the 2016 and 2019 assumptions for these three variables are shown in the graphs below. The current assumptions are reasonable. Other economic data used to prepare Budget 2019 are discussed in the various sections pertaining to each of the customer classes. The historical hourly load profiles for 2015-2017 are included in Attachment 9.2-1 below.

Attachment 9.2-1: Hourly Load Profiles (MW) for 2015-2017

# 9.3	ACTUAL & EXPECTED INTERRUPTIBLE DEMAND

Attachment 9.3-1: Budget 2019 Actual & Expected Interruptible Demand

# 9.4	EVALUATION OF PREVIOUS FORECASTS

This section describes an evaluation of the last four forecasts developed over the past five years. This includes Budget 2015, Budget 2016, Budget 2017, and Budget 2018. This section will show comparisons of actual and weather normalized energy and demand to the various Budgets. This section will also give a qualitative assessment of the general changes that were made in the development of each forecast and the specific major changes that have been made in the development of recent forecasts.

Georgia Power monitors the performance of the current forecast on a monthly basis. This is done through a forecast variance report. The monthly forecast variance report compares the forecast of class sales for the current month to the actual class sales of the current month. This report also identifies how much of the energy variance is a result of the deviation between the actual weather conditions for the month and those used in the forecast. Actual and weather normalized actual sales (by class) are compared to the energy forecast. These comparisons are made for the billing month and calendar month for each class. Reports of this sort help detect deviations from the forecast and help identify discrepancies that may be systematic.

Certain changes are made from year to year in response to evaluating previous forecasts, incorporating additional information, and meeting new or enhanced informational needs. These changes are generally described in two categories: (1) general activities and procedures used to produce new forecasts and (2) specific enhancements made to improve the forecasts.

Attachment 9.4-1 shows comparisons of actual and weather adjusted (weather normalized) energy versus budget by class of service for Georgia Power. The report contains comparisons for residential, commercial, industrial, street lighting, MARTA, retail, and territorial.

Attachment 9.4-1: Energy Comparisons by Class (Calendar data)

Attachment 9.4-2: Actual and Budget Peak Demand (Calendar data)

Attachment 9.4-3: Comparison of Forecast and Actual Demand

Attachment 9.4-4: Comparison of Forecast and Weather Normal Demand

## 9.4.1	General Enhancements

Whenever possible, each new forecast incorporates changes in a variety of areas that will improve the quality of the forecast. New and revised data relating to the economy, demographic conditions, and fuel, for example, are included. In addition, existing models are enhanced or are replaced with new models that take advantage of better data or improved estimation techniques. Model specification, parameters, and equations are reviewed, assessed, and changed as appropriate.

Data:  Each energy and demand forecast uses a recent, comprehensive set of consistent economic and demographic variables such as income, households, commercial floor space, housing stock and employment. The latest available forecasts of the prices of alternative fuels (generally gas and electricity) and the latest available saturation data are also incorporated where appropriate. End-use device standards are reassessed and updated as available information permits. Market profile data from survey and end-use research are assembled and used as newer or better data become available. Weather files are updated annually.

Models:  Model specifications are reviewed and adjusted as necessary to account for new products, data availability, and significant events that may affect model accuracy. New explanatory variables may be added to the short-term econometric models to capture recent changes, while variables that no longer explain the variable being forecasted are removed.

Model Coefficients: Whenever new data are available or model specifications change, the forecasting equations are re-estimated, and new model coefficients are generated.

Judgment:  Based on new research, information, and experience, the collective expertise available to the Company is used to refine model results to produce the best possible forecast.

## 9.4.2	Specific Enhancements

Research, new information, and reviews of previous forecasts often reveal findings that enable the Company to make improvements to the forecast. The changes identified below chronicle the evolution of the Budget 1998 forecast through the current Budget 2019 forecast. Budgets are typically completed in the Fall of the previous year (Budget 2019 was completed during Fall 2018). The major changes are identified below:

Budget 1998

Modeling of the economic and demographic forecasts and production of the economic forecast for the state of Georgia were outsourced to Regional Financial Associates. Data Resources Incorporated provided the national economic forecast.

The existing commercial and industrial models were replaced by EPRI models, COMMEND and INFORM.  This was done to maximize resource efficiency, minimize costs, and maximize resource expertise.  Forecasts for all three major energy classes were developed using EPRI products (REEPS for residential).  The EPRI products were well recognized nationally throughout the utility industry and employ consistent frameworks in each model while specifically targeting the necessary elements unique to each class.

Budget 2000

Trend-regression (econometric) methods used to develop residential, commercial, and industrial short-term energy models.

Budget 2001

Consistent 15-year weather normal definition adopted for both the econometric (short-term) and end-use (long-term) models for the energy sales forecast.

Adopted HELM to determine peak demand weather normalization, beginning with 2000’s peak.   Regression model used for years prior to 2000.

Updated HELM with new class load data and included multiple years in the development.

Budget 2004

Normal weather definition in HELM changed from a 30-year average to a 15-year average, making HELM normal weather consistent with the energy models.

Budget 2006

Updated the RTP response model.

Made adjustments in data to capture continuing customer reclassification.

Budget 2007

Energy histories further adjusted for customer reclassification.

Transmission and distribution loss factor methodology updated.

Savannah merger impacts:

Energy histories updated

Weather history updated

REEPS, COMMEND and INFORM and HELM updated

Budget 2009

Added price variables to short-term energy models.

Budget 2010

Replaced industrial Gross State Product (“GSP”) with industrial production to measure output in the short-term industrial energy models. Replaced 15-year weather with 20-year weather to improve forecast consistency across operating companies.

Budget 2011

In the long-term residential model, added several new end uses, including computers, ceiling fans, DVRs, pool pumps, well pumps; split lighting into two types (traditional and fluorescent); split television into 2 types (conventional and other technologies such as plasma & LCD).

In the long-term commercial model, revised base year EUIs for education and warehouses due to addition of some base year share for office equipment. Added office occupancy rate as an exogenous variable to capture downturn in the economy.

In the residential customer model, the number of households variable was replaced with housing stock and housing starts to capture new housing activity.

A number of changes were made in the short-term commercial model. The data range was shortened to begin in 2000 rather than 1990. Also, the interaction of the weather variables (heating and cooling degree days) with square footage was dropped because of the increase in the commercial vacancy rate associated with the Great Recession. The billing days variable was dropped, and Atlanta population was added as a variable to capture new business formation.

In the short-term industrial model, cooling degree days interacted with monthly binaries replaced individual monthly dummy variables.

Budget 2012

In the long-term residential model, the following changes were made:

- Base year was changed from 1999 to 2007

- Vintages for shares/size/efficiency were updated so that prior to 2005 was “old” and 2005 to current was “new”

- Base year “existing” shares obtained from 2007 Saturation Survey

- Penetration “new” shares obtained from 2010 Saturation Survey

- Changed UEC source (for all end uses except HVAC) from national RECS data to EnerSim results run in-house, in order to get a more realistic picture of usage specific to the Georgia Power service territory

- Split COOK into two categories (oven and stovetop)

- Split water heating from two to five end uses (electric resistance, electric HP, electric tankless, gas, gas tankless)

- Updated income and home size elasticities

In the residential customer model, the data range was shortened to begin in 2000 rather than 1990. The unemployment rate replaced housing starts as an economic variable.

Budget 2013

Historical data ranges were shortened in the short-term residential, commercial, and industrial models. For residential, the data for the new model began in 2002 rather than 1999, and for commercial and industrial, it began in 2005 rather than 2000. In the industrial energy model, manufacturing employment was dropped.

Budget 2014

The new long-term energy model (LoadMAP) and peak model (PDM) were run in parallel with the previous models, REEPS, COMMEND, INFORM, and HELM.

In the short-term energy models, the sample periods over which the models were estimated changed. In the residential model, the sample period began in 2002, rather than 2000 as was the case in the Budget 2013 forecast. The commercial and industrial sample periods were lengthened, with commercial beginning in 2001 rather than 2005, and with industrial beginning in 2000 rather than 2004.

Other changes included dropping the humidity variable from the residential use-per-customer model and using the Georgia Industrial Production Index instead of the Georgia Industrial Production Index for Manufacturing.

Budget 2015

REEPS, COMMEND, INFORM and HELM were not used. LoadMAP and PDM became the official long-term energy and peak models. LoadMAP models used 2007 as the base year. Load shape representations used in the PDM were from 2010 and 2011.

The sample periods for the commercial and industrial models were extended back one year to 2000 and 1999, respectively. The autoregressive term that had been included in the commercial model previously was no longer necessary. In the industrial model, the U.S. Industrial Production Index replaced the Georgia Industrial Production Index used in Budget 2014, and manufacturing employment was included as an additional driver.

Budget 2016

The short-term residential use-per-customer model underwent two significant changes. The first was that the summer weather variables are no longer divided by the SEER index. The second change was that real personal income was replaced as a driver by the total housing stock in Georgia. A strong forecast of real personal income resulted in unrealistic growth rates for residential use-per-customer. Thus, real personal income was not used as a driver in the model.

The short-term commercial energy forecast model also underwent significant revisions for Budget 2016. The key economic driver of the commercial model is total Georgia population. This replaces Atlanta population and real Georgia Commercial GDP, which had been used as drivers in previous years. Lastly, the beginning year of the sample period was changed to 2001 from 2000.

The short-term industrial energy forecast model used the Georgia Industrial Production Index instead of the U.S. Industrial Production Index.

The RTP response model was updated by Christensen Associates Energy Consulting, LLC.

Budget 2017

In Budget 2017, the ratio of multi-family housing stock to total housing stock was used to capture the lower energy usage of the growing multi-family sector.

Budget 2018

Budget 2018 included updates to the weather variables used in the short and long-term energy forecast models. The number of weather stations used and the weighting of each station; the breakpoints used to calculate HDH and CDH; and the definition of normal weather were updated with recent historical data to capture changing customer trends.

There were three changes to the short-term forecast models. First, an energy efficiency variable was included in the residential and commercial forecast models, as suggested by the Georgia Public Service Commission (“Commission”) staff in the 2016 IRP. Second, the ratio of multi-family housing stock to total housing stock was dropped as a driver of residential energy usage. Lastly, the number of Georgia households replaced population as a driver of residential customers and commercial energy.

There were also a few changes to the long-term and peak demand forecast models. Navigant lighting saturation assumptions were adopted in the residential, commercial and industrial LoadMAP models. The load shape representations used in the peak demand model were re-estimated using actual 2015 and 2016 load data.

Budget 2019

Changed economic service providers from Moody’s Analytics to IHS Markit following the issuance of a Request For Proposals in 2017. IHS Markit became Georgia Power’s provider of economic data and forecasts in January 2018 and these economic forecasts were used to produce Budget 2019.

The short-term econometric forecast models were tested for structural breaks and it was determined that the estimation periods needed to be shortened to better capture the changing relationships impacting energy use. The time period used to estimate the residential model begins in January 2011; the commercial model begins in January 2009; and the industrial model begins in January 2010. In addition, electricity prices were excluded as a driver in the short-term energy models, although a price variable remains in the long-term models. Lastly, the short-term commercial model no longer includes households as a driver of energy use, and the short-term industrial model no longer includes the Georgia Industrial Production Index as a driver.

The Commercial LoadMAP model was updated to better capture the impacts of energy efficiency and customer behavior. Specifically, the model used updated technology specifications, such as EERs and unit energy consumption, throughout the end-use categories.

The RTP response model was updated by Christensen Associates Energy Consulting, LLC.

# 9.5	ENERGY RESEARCH PLAN

The following sections contain a description of Georgia Power’s Energy Research Plan for 2019-2022. This plan will allow Georgia Power to meet the Commission’s requirements, including required filing of information, development of forecasts, and evaluation of demand and supply side resources.

## 9.5.1	Introduction

In compliance with the Commission’s rules, Georgia Power submits its four-year Research Plan, 2019-2022. This plan contains three major sections:

1. 	Survey Research

2.	Load Research

3.	Analysis

The research program addresses specific requirements of the Commission rules. It includes only that research which is designed to meet specific filing requirements or is associated with meeting data requirements to support the filing of an integrated resource plan. Research programs associated with evaluation plans for demand side program certification are contained in the respective program filing. Table 9.5.1-1 summarizes the specific sections of the rules, and the proposed research program.

Table 9.5.1-1: Research Requirements for Complying with Commission Rules 515-3-4

| Section | Requirement | Research Program |
| --- | --- | --- |
| 515-3-4.03 (1) - (2) (b) Energy and demand forecasting requirements | Historical peak demands, load factors, energy consumption by class. | Perform a continuous class load research study. |
| 515-3-4-.03 (3) (a) Forecast methodology | Most current data on end-use penetration and saturation; penetration and saturation of the market of competing fuel end uses; behavioral factors affecting energy use. | Identify and incorporate secondary sources of data and behavioral factors affecting energy use. |
| 515-3-4-.03 (3) (c) Data requirements | Develop a data base of electricity consumption patterns by customer class and end use. | Maintain and leverage existing end-use load shape information with Energy Simulation tools. |
| 515-3-4-.03 (3) (e) | Identify and describe ongoing and planned load research | Continuous load research studies of all customer classes. The quality of each sample is checked periodically, and the sample is replaced if needed. |
| 515-3-4-.04 (4). Potential new demand side resources | Data to support analysis of new demand side resources, including identification of customer needs, and assessment of market potential. | Evaluate energy efficiency measures that are suitable for Georgia Power customers, climate, residences, buildings, and facilities. Conduct an energy efficiency technical, economic, and achievable potential study. |

## 9.5.2	Survey Research

### 9.5.2.1  Regulatory Requirements

Commission rules require the filing of information on the saturation and penetration of end-use equipment, including both electrical and alternative fueled end uses. In addition, the rules require that the forecast be based on a disaggregated end-use method or some other comparable forecasting methodology, which requires information on the saturation and penetration of end-use equipment. Further, the rules require the development of an end-use technology catalog based on service area specific information, including information on equipment efficiencies, useful life, and energy and capacity impact information. To the extent that the disaggregated end-use methodology is used to develop the forecast, this research program will provide the data necessary to support this analysis and development of the forecast.

### 9.5.2.2  Data Collection and Utilization Plan

Use readily available information such as the United States Department of Energy’s Energy Information Administration RECS, Commercial Building Energy Consumption Survey (“CBECS”) and MECS for the South Atlantic Region to develop end-use penetration and saturation trends and measure customer building characteristics.

## 9.5.3	Load Research

### 9.5.3.1  Regulatory Requirements

Commission rules require the filing of information on the demand and energy characteristics of each of the utility's classes, including load profiles, on-peak, off-peak, and shoulder kWh, and summer, winter and annual peak demands, for the most recent three years of history and the first three years of the forecast period.

The rules require the development and filing of load and consumption information for each customer class.  In addition, the rules require that the forecast used in the Integrated Resource Plan filing be based on disaggregated end-use methods or some other comparable forecasting methodology.

The load research component of this research plan will provide the data to meet these requirements.

### 9.5.3.2  Class Load Research Project

Georgia Power uses continuous load studies of the residential, commercial and industrial classes that include a series of studies designed to the rate/class level. Over 2,000 customers are selected for these studies. The studies have a combination of census groups for small populations and statistical samples for larger customer groups served at secondary distribution. These studies have installed load metering equipment on a sample of customers from each rate/class combination. Combined with over 3,000 customers with interval metering equipment used for billing, the studies provide half hour data for each rate/class combination.

## 9.5.4	Analysis

### 9.5.4.1  Regulatory Requirements

Commission Rule 515-3-4 has the following requirements that will continue to be met through analysis of data collected in the preceding described research projects and/or review of existing currently available data from other sources:

Development of a data base of electricity consumption patterns by customer class and end use. (Commission Rule 515-3-4-.03 (3) (c))

### 9.5.4.2  Analysis Projects

#### 9.5.4.2-1  End-Use Load Shape Catalog

Georgia Power, in support of the development of the technical potential study, develops a catalog of end-use load profiles. These profiles are based on engineering studies that are supplemented with the best available internal and external end-use load research. Additionally, Georgia Power conducts an annual rate and class load research study that is used in the Company’s cost of service studies and forecasts.

Going forward Georgia Power plans to continue to update the end use load catalog as needed to support future Technical Potential Studies and continue its annual load research program at the rate and class level.

#### 9.5.4.2-2  End-Use Load Shape Modeling

An energy simulation tool is being used to develop end-use and total premise load shapes based on both previously collected empirical data and engineering simulation.  It is an engineering-based simulation tool that enables the user to develop energy use profiles and load shapes for various building types and energy system combinations.

# 10.0 ACRONYM & ABBREVIATION GLOSSARY

| Acronym | Definition |
| --- | --- |
| AAGR | Average Annual Growth Rate |
| AHAM | Association of Home Appliance Manufacturers |
| AEG | Applied Energy Group |
| BTM | Behind-The-Meter |
| BTU | British Thermal Unit |
| Budget 2019 | Budget 2019 Load and Energy Forecast |
| CAGR | Compound Annual Growth Rate |
| CBECS | Commercial Building Energy Consumption Survey |
| CDH | Cooling Degree Hours |
| Commission | Georgia Public Service Commission |
| CP | Coincident Peak |
| CRA | Charles River Associates |
| CVR | Conservation Voltage Reduction |
| DEP | Distribution Efficiency Program |
| DLC | Direct Load Control |
| DOE | U.S. Department of Energy |
| DPEC | Demand Plus Energy Credit |
| DSM | Demand Side Management |
| DSO | Demand Side Options |
| EIA | Energy Information Administration |
| EISA | Energy Independence and Security Act of 2007 |
| EPACT | Energy Policy Act of 2005 |
| EPRI | Electric Power Research Institute |
| EUI | Energy Usage Index |
| GDP | Gross Domestic Product |
| GSP | Gross State Product |
| HDH | Heating Degree Hours |
| HVAC | Heating Ventilation & Air Conditioning |
| LoadMAP | Load Management Analysis and Planning |
| LT | Long Term |
| MARTA | Metropolitan Atlanta Rapid Transit Authority |
| MECS | Manufacturing Energy Consumption Survey |
| NAECA | National Appliance Energy Conservation Act of 1987 |
| NAICS | North American Industry Classification System |
| Acronym | Definition |
| NCP | Non-Coincident Peak |
| NCTA | National Cable and Telecommunications Association |
| NEPA | National Energy Policy Act of 1992 |
| PDM | Peak Demand Model |
| RECS | Residential Energy Consumption Survey |
| RTP | Real Time Pricing |
| SBG | Stand-by Generation |
| SCS | Southern Company Services |
| SEER | Seasonal Energy Efficiency Ratio |
| SIC | Standard Industrial Classification |
| ST | Short Term |
| TOU | Time of Use |
| UEC | Unit Energy Consumption |
| UI | Usage Index |

On January 1, 2019 Southern Company sold Gulf Power Company to NextEra Energy. However, for purposes of Budget 2019, Gulf Power Company was included in the Company’s assumptions and evaluated alongside the other Southern Company operating companies as a member of the pool.