STF-9-2

Question:

In reference to page 35 lines 18-20, with respect to system lambda, specifically to the lines stating that a unit “dispatched for purposes of testing and not for serving the next increment of load…does not set lambda.”:

- Please describe, in detail, the process used by the Company to determine the commitment status of long-lead time units, including considerations, if any, of fuel cost, variable O&M cost, emissions cost, system lambda forward projections, startup / shutdown cost(s), and other considerations.

- To the extent that the Company uses a spreadsheet or calculation to determine commitment status of long-lead time units, please provide an example of this spreadsheet or calculation from the first week of each month in 2019 and 2020, or a similarly representative selection. If the Company does not use a spreadsheet mechanism, provide an example of the process used with as much granular detail as possible.

Response:

- The Fleet Optimization group uses a production cost model that is an internally developed application utilizing a commercially available solving engine. The production cost model considers items such as unit variable costs, startup costs, minimum up and down times, ramping capabilities, must-run requirements, and unit operating limits, combined with the load forecast and reserve requirements, to provide a commitment plan that minimizes production cost and meets the instantaneous load and reserve requirements subject to operating constraints.

- The model provides the least cost commitment solution for each hour of the next ten days. This commitment plan is analyzed by the Fleet Optimization group with consideration of additional known system constraints to formalize the next day’s commitment plan. The Fleet Optimization group commits the system six days per week; on Friday each week, optimization is run for the following Saturday and Sunday. As the calculations are performed within an internal model, there are no spreadsheets to provide and no reports detailing the calculations.