Page Number 1.70

GEORGIA POWER COMPANY

Rules and Regulations for Electric Service

- CUSTOMER GENERATION:

  - All customers who intend to install a customer generation facility at their premises shall be required to execute an applicable Interconnection Agreement prior to being allowed to interconnect. The customer is responsible for ensuring a safe and reliable interconnection with the Company system and all costs incurred therein.  The Company reserves the right to disconnect from its lines or refuse to connect to its lines any customer with customer generation that does not comply with the requirements and safety standards set forth in Section F4.  Customer must adhere to the guidelines set forth in the “Southern Company Distribution Interconnection Policy” or its successor, which establishes criteria for interconnection including system protection, power quality, and other operational requirements.

  - All customers on a Company distribution circuit who desire to install customer generation behind their meter must adhere to the Company’s interconnection process and operation requirements which are detailed in the “Behind-the-Meter Distribution Interconnection Summary” or its successor, available to the public on the Company’s website and from Company representatives.

  - All customer generation sized 250 kilowatts and above, whether installed behind-the-meter or stand alone, must undergo a system impact study and the witness testing process as required by the Company and will be responsible for all associated fees.  This customer generation must also have a local communications interface, installed, and maintained by the customer, that supports information exchange with, and control by, the Company.  This requirement does not apply to end-use customer owned emergency or standby generators that create momentary parallel operation with Company’s electric system for no more than 100 milliseconds.  Customer-owned generators less than 250 kilowatts may also be required to undergo witness testing if the Company deems that the generator poses a safety or reliability risk.

  - All customers proposing to interconnect a customer generation facility shall pay a one-time interconnection fee of $200. The interconnection fee will cover the cost of processing project applications for interconnection according to the Company policies and the applicable agreement, including but not limited to costs for (a) initial interconnection feasibility review and identification of potential impacts to Company lines or equipment; (b) applicable metering and/or meter programming; (c) internal technology systems, software and applications specifically used to process interconnection of customer generators; and (d) system witness testing (as necessary) for generators under 250 kilowatts.