STF-JKA-1-11

Question:

Regarding interconnection and build limits modeled for generic resources:

- For all the modeled portfolios, provide any annual and cumulative build limits that were modeled by resource. Also, provide all workpapers used to derive the limits as used in Aurora modeling in the IRP.

- Has Georgia Power changed any annual or cumulative build limits between the 2023 IRP Update and the 2025 IRP? Explain what has changed and why, if applicable.

- Provide supporting industry information, justification, and support for the interconnection and/or build limit assumptions utilized.

- Please describe any firm transportation and/or gas availability assumptions considered in the development of the limits modeled.

- What contractual commitments or financial commitments, if any, has Georgia Power/SCS/Southern Company made to Transco for Southeast Supply Enhancement (SSE) project as of January 15, 2025?

- What contractual commitments or financial commitments, if any, has Georgia Power/SCS/Southern Company made to SNG for South System 4 (SS4) project as of January 15, 2025?

- What contractual commitments or financial commitments, if any, has Georgia Power/SCS/Southern Company made to Transco for FT on Station 85 receipt area (West Alabama) northeast to Georgia as of January 15, 2025?

Response:

- See 2025 IRP Technical Appendix Vol 2 Resource Mix Study for all modeled annual and cumulative build limits. Additionally, see STF-JKA-1-11 Attachment A TRADE SECRET for derivation of the natural gas firm transportation (“FT”) availability limit used for expansion CC resources.

- Table 1 below summarizes the build limits assumed in 2025 IRP capacity expansion modeling and provides a comparison with assumptions in the 2023 IRP Update.

Table 1. Summary of Limits on Expansion Units.

| Technology | 2023 IRP Update | 2025 IRP |
| --- | --- | --- |
| CT w/SCR | - No limit | - No limit |
| CC | - Cumulative build limit based on annual FT availability | - Cumulative build limit based on annual FT availability |
| CC w/CCS | - Cumulative build limit based on annual FT availability | - Cumulative limit based on annual FT availability - Distinguishes between Local and Distant CCS - Local CCS limited to REDACTED total (REDACTED limit for GPC) |
| Solar | - Annual Max – 1,500 MW/yr | - Annual Max – 1,500 MW/yr |
| Wind | - Annual Max – 600 MW/yr - Cumulative Max – 8,100 MW | - Annual Max – 300 MW/yr - Cumulative Max – 4,500 MW |
| BESS | - Annual Max – 3,000 MW/yr - Cumulative Max – 18,000 MW | - Annual Max – 3,000 MW/yr - Cumulative Max – 18,000 MW |
| MDESS | - No limit | - No Limit |
| Nuclear | - Annual Max – 300 MW/yr | - Annual Max – 600 MW/yr |

### Annual Limit on CT and CC Builds

The Company used an annual maximum build limit in the 2023 IRP Update for CTs and CCs that was calculated based on the system’s annual incremental capacity need.  The purpose of this limit was to reduce the number of new resource options Aurora evaluates and improve runtime.  Because the limit was set at or above the system’s annual incremental capacity need, it did not actually limit the number of CTs or CCs that were selected by the model.  For the 2025 IRP, the Company determined that having a static value that is large enough to allow Aurora to meet an entire year’s capacity need with a single new resource type (if optimal), but not calculated based on annual need, simplifies the calculations required for each scenario and does not significantly increase run time.

For CTs and CC w/CCS, this limit is set to 30 blocks per year, which is equivalent to 9,000 MW (300 MW/block).  For CCs, the build is limited to 3 blocks in 2029, the first year CC is allowed, and 15 blocks in 2030 to represent the estimated amount that could be constructed in that timeframe.  For 2031 and beyond, the CC build limit is set to 30 blocks per year.

### FT Availability Limit on CC Builds

In both the 2023 IRP Update and the 2025 IRP, the Company assumed FT used for existing units would become available to new CCs as the existing units either retired or power purchase agreement (“PPA”) expired.   In addition, for planning purposes, the Company assumed some additional pipeline expansion and associated FT would become available during the planning horizon. STF-JKA-1-11 Attachment B TRADE SECRET shows the resulting cumulative MW build limits that were included in the capacity expansion modeling for the 2023 IRP Update and the 2025 IRP.

### Assumptions on Gas Availability for CTs

Given the current operational realities and the challenges facing the pipeline infrastructure industry, the Company assumes the most reliable operational plan for new generic expansion CTs is for them to be dual-fueled, capable of combusting either fuel oil or natural gas.  For the 2023 IRP Update, the Company assumed year-round fuel oil operation for new generic CTs because the pipelines serving the Company region have become increasingly constrained and less flexible in recent years. For the 2025 IRP, this assumption was revised to only require CTs to operate on oil during the winter months (Dec-Jan-Feb) and to operate as needed on gas throughout the remainder of the year.

### Local Versus Distant CC w/CCS

Areas of favorable geology for carbon storage within the Southern Company footprint are limited.  For CCs that are distant from carbon storage regions, application of CCS requires that the captured CO2 be transported by pipeline to a storage site (i.e., Distant CC w/CCS), which results in significant additional cost to construct and operate the unit.   For the 2025 IRP, the Company has determined that it is reasonable to assume that only REDACTED of CC w/CCS could be constructed near favorable carbon storage sites (i.e., Local CC w/CCS), with significantly less cost for CO2 transport than Distant CC w/CCS.  The Company assumes REDACTED miles of pipeline for Local CC w/ CCS and REDACTED miles of pipeline for Distant CC w/ CCS.  In addition, the Company assumes that only REDACTED of Local CC w/CCS could be constructed in the GPC service territory. Local CC w/CCS builds in the capacity expansion modeling are limited based on operating company need (e.g., if GPC is the only Operating Company with a capacity need, the build cannot exceed GPCs need for that year and cannot exceed REDACTED) and is limited to a system total of REDACTED.  This limit was not included in the 2023 IRP Update modeling.

- The Company aims to develop any generic resource limits tailored to technology deployment within its regulated service territory, rather than relying on public sources, which often cover a broader geographic scope. It is common utility practice to apply limits on generic resources in expansion plan modeling.

- Given the challenges facing the pipeline infrastructure industry, the Company has applied a limit on the amount of new CC (including CC w/CCS) builds allowed in capacity expansion modeling. The limit is a cumulative build constraint that is based on the amount of natural gas firm transportation (“FT”) that is estimated to be available from retirement of existing resources, PPA expiration, and/or pipeline expansion.  See JKA-1-11 Attachment A TRADE SECRET for derivation of the FT limits used for CC expansion resources.

- Please see the response to STF-JKA-1-13.

- Please see the response to STF-JKA-1-13.

- Georgia Power/SCS/Southern Company have not made any contractual or financial commitments to Transco for new capacity with receipt points in the Station 85 area.