STF-JKA-1-27

Question:

How did Georgia Power address the Commission approved Stipulation/Order from the 2022 IRP, specifically regarding the following paragraphs. Also for each item below, explain how the Company modeled the resources in the Company’s Load and Resource Balance Table, and in Aurora:

a. Stipulation par. 19 – 2,100 MW of new utility scale renewable resources associated with the CARES program, greater than 6 MWs in size.

b. Stipulation par. 22 – Up to 200 MW of distributed generation solar resources between 250 kW and 6 MWs in size.

c. Stipulation par. 27 – The Community Solar Program.

d. Stipulation par. 33 – 250 MW pilot Demand Response Credit Tariff and the Resilience Asset Service Tariff.

e. Stipulation par. 35 – The Tall Wind demonstration project.

f. Commission Order par. 73 – Georgia Power’s acquisition of 500 MW of additional Energy Storage Systems via the Commission’s RFP bid process.

Response:

See Chapter 2, “Implementation of Prior IRPs,” in the 2025 IRP Main Document for discussion regarding implementation of the 2022 IRP.

- The 2,100 MW of new utility scale renewable resources associated with the Clean and Renewable Energy Subscription (“CARES”) Program referenced in Stipulation para. 19 is shown in the Company’s Load and Resource Balance Table as “2022 IRP SOLAR US RFP 1” and “2022 IRP SOLAR US RFP 2”. In Aurora, the program is modeled as “O1_2022_IRP_US_RFP_1” and “O1_2022_IRP_US_RFP_2”.  The modeled nameplate capacity for the 2022 IRP SOLAR US RFP 1 was reduced to 50% from the 2023 IRP Update based on expected procurement challenges. See Chapter 8 Section 11.2 on Dynamic Procurement Challenges for Renewable Procurement of the 2025 IRP Main Document for more information.  Additionally, the first year of availability for both resources was delayed in the model. Based on market feedback, the RFP was modified to allow for RCODs as late as 2029.

- The distributed generation solar resources referenced in Stipulation para. 22 of the 2022 IRP are shown in the Company’s Load and Resource Balance Table as “2022 IRP SOLAR DG RFP 1” and “2022 IRP SOLAR DG RFP 2”. In Aurora, the program is modeled as “O1_2022_IRP_DG_RFP_1” and “O1_2022_IRP_DG_RFP_2”.

- The Customer Solar Program referenced in Stipulation para. 27 of the 2022 IRP is shown in the Company’s Load and Resource Balance Table as "Community Solar - Comer", "Community Solar - Guyton", and "Community Solar - Waynesboro". In Aurora, the program is modeled as “O1_COMMSOLAR_COMER”, “O1_COMMSOLAR_GUYTON”, and “O1_COMMSOLAR_WAYNESBORO”.

- The Demand Response Credit Tariff and the Resilience Asset Service Tariff pilot program referenced in Stipulation para. 33 is shown in the Company’s Load and Resource Balance Table under Dispatchable DSO Programs as “DER Customer Programs.” It is modeled as 0 MW of capacity because there are currently no active customers enrolled in the program.

- The Tall Wind demonstration project referenced in Stipulation para. 35 was not modeled in the Load and Resource Balance Table or Aurora. As set forth in Paragraph 74 of the approved 2022 Order, the Commission did not approve the provision of the Stipulation for the tall wind demonstration project.

- The 500 MW of energy storage system (“ESS”) resources approved in the 2022 Integrated Resource Plan (“IRP”) was for a request for proposals (“RFP”) to the market and is shown in the Company’s Load and Resource Balance Table as “2022 IRP ESS RFP” and modeled in Aurora as “O3_2022_IRP_ESS.”