STF-JKA-1-31

Question:

How did Georgia Power address the Commission approved Stipulation from the 2023 IRP Update, specifically regarding the following paragraphs. Also for each item below, explain how the Company modeled the resources in the Company’s Load and Resource Balance Table, and in Aurora:

a. Par 7 – 500 MWs of BESS at various sites on the system (Par 8 states Robins and Moody was included in this).

b. Par 9 - 500 MWs of additional BESS resources through an expedited RFP.

Response:

See Chapter 2, “Implementation of Prior IRPs”, in the 2025 IRP Main Document for discussion regarding the Company’s implementation of the 2023 IRP Update.

- A table of these projects is provided below along with their corresponding Aurora Name, Commercial Operation Date (“COD”), Nameplate Capacity (MW), and Reliable Capacity (MW). Robins and Moody Batteries are limited to the transmission delivery service amount at Robins Solar and Moody Solar, respectively. Therefore, the reliable capacity is the associated solar facilities winter planning capacity value (nameplate capacity multiplied by winter Effective Load Carrying Capability (“ELCC”) percentage) subtracted from the nameplate capacity. This practice ensures that the Company is not overstating the capacity granted at the point of interconnection.

| Project Name | Aurora Name | COD | Nameplate Capacity (MW) | Reliable Capacity (MW) |
| --- | --- | --- | --- | --- |
| Hammond | O3_HAMMOND_BESS | 2026 | 57.5 | 57.5 |
| McGrau Ford Phase 2 | O3_MCGRAU_FORD_PHASE_2 | 2026 | 265 | 265 |
| Robins | O3_ROBINS_AFB_BESS_4hr | 2026 | 128 | 115 |
| Moody | O3_MOODY_AFB_BESS_4hr | 2026 | 49.5 | 45 |

- The resource is shown in the Company’s Load and Resources Balance Table as “2023 IRP UPDATE BESS RFP” and modeled in Aurora as “O3_2022_IRP_ESS.” Capacity needs are currently expected to exceed the 500 MW projected during the 2023 IRP Update. The Company’s Resource Mix Study does not reflect any assumed capacity procurement amount associated with the Winter 2027/2028 BESS RFP. This is to show the total procurement required from those RFPs to meet capacity needs from the winter of 2027/2028 through the winter of 2030.