STF-JKA-1-8

Question:

Provide source documents for any capital cost escalation rates that were modeled.

Response:

All technologies except for wind, solar, and battery storage are escalated at the rate of the Gross Domestic Product Implicit Price Deflator (“GPDIPD”). Please refer to attachment JKA-1-8 Attachment A TRADE SECRET for development of the Implicit Price Deflator used to model generic units in the Resource Mix Study.

Escalators for solar and battery storage, as represented in Figure 12 of the Resource Mix Study in Technical Appendix Volume 2, were developed using capex forecast data from the “1Q 2024 Global PV Market Outlook” from Bloomberg New Energy Finance (“BNEF”). The escalator for wind is based on similar data from the “2024 v1 Annual Technology Baseline” (“ATB”) from the National Renewable Energy Laboratory (“NREL”). The calculation of these escalation rates begins with the overnight capital cost forecasts for each technology from the most recent BNEF Levelized Cost of Energy (“LCOE”) spreadsheet that can be accessed by BNEF clients. When these escalation trajectories were established, the most recent BNEF LCOE publication was the “Energy Storage System Cost Survey 2023.” Solar PV was informed by BNEF while wind was informed by the ATB. The forecasted values published by BNEF were originally in 2023 dollars. To be consistent with other costs included in the Resource Mix Study, nominal values were calculated for this forecast by escalating each year’s projection in real terms using the Company’s forecast of the GDPIPD. The final step normalizes the forecasted trajectories so that the capital cost changes can be applied to the Company’s most recent overnight cost estimate in the first year of the forecast. The normalization calculation divides the nominal capital cost projection for all future years by the overnight cost estimate in year one. The normalized capital escalation trajectories for onshore wind, solar PV and battery energy storage are included in the “Mix Study - Figure 12 Normalized Wind Solar BESS Projections TRADE SECRET.xlsx” workpaper provided to Commission Staff on January 31, 2025, in the Technical Appendix Vol 2 – Resource Mix Study workpapers folder. Development of the chart provided in the workpaper is detailed in STF-JKA-1-8 Attachment B TRADE SECRET.

The Company uses the BNEF capital cost forecasts included in its LCOE publications because it is driven by extensive data collection and quantitative learning curve analysis by BNEF as described in the Methodology section of the Energy Storage System Cost Survey 2023, which is accessible to BNEF clients.