STF-JKA-2-16

Question:

Regarding the URS workpapers that were provided for Bowen, Scherer, and Gaston. When workpapers are requested below, provide such workpapers electronically with all formulae intact.

        - Provide an overview of the important tabs in the URS study workpaper and explains how the tabs work together to derive the final study results.

        - Provide additional workpapers used to derive the Amortization tab as all values are pasted. Also, provide a narrative explanation of the derivation of the Declining Revenue Requirement Assumptions, and the Equivalent Factor.

        - In the URS analysis explain how Declining Revenue Requirements are used and how revenue requirements expressed as Economic Carrying Charges are used.

        - Why is the Company using the Stale Value Formatting feature as it was not use in past IRPs.  What is the Company’s goal for using that feature, as it requires constant manual recalculations as one moves between and within tabs.

        - Refer to the tab – “Cost Inputs” for each of the URS studies. Provide the source workpapers and supporting documentation building up all of the assumed costs (Maintenance Capital, Fixed O&M, Environmental Capital, Environmental O&M, Lateral Capital, FT, Conversion Capital, 45Q).

        - Provide all workpapers used to go from Capital Costs to revenue requirements associated with the capital cost items referred to in part d above.  In past IRPs, the Company supplied Samlite workpapers. Were those workpapers developed in the 2025 IRP, but not supplied?  If so, please supply all additional workpapers not yet supplied.

        - Refer to the tab – “Transmission” and explain why the revenue requirements are included on a separate tab apart from the Cost Inputs tab, like the other capital cost items.

        - Refer to the tabs – GenericProdCost and Energy – Please provide all workpapers, including Aurora models, if not already provided, that were used to derive the values pasted in these tab.  Also, explain the methodology used to derive the values included in those tabs.

Response:

- “Study” Tab – The “Study” tab allows you to define the studies, which align with the environmental compliance pathways. These study names flow through to other tabs, which allow for cost input, capacity input, energy input, results, etc. This is also where the following items are defined: the study period, the replacement unit, capacity equalization MW, firm transportation (“FT”) rate for the generic replacement unit, and scaling of replacement unit.

“CapacityNeeds” Tab – The “CapacityNeeds” tab allows you to define the capacity need for that specific study or environmental compliance pathway. These values are used to calculate Term Equalization on the “Capacity” tab.

“Capacity” Tab – This tab allows the user to define capacity associated with each study or environmental compliance pathway. It then assigns a Term Equalization cost for each Scenario. If there is a capacity need, which is pulled from the “CapacityNeeds” tab, and the capacity is not provided by the units in that study, then a Term Equalization is calculated. The Term Equalization is calculated using the net marginal cost of a combined cycle (“CC”), which is pulled from the “GenericFixedCost” tab and the “GenericProdCost” tab.

“Cost Inputs” Tab – The “Cost Inputs” tab allows the user to input all costs associated with that study or environmental compliance pathway. Column E tells the spreadsheet how to sum the costs in summary tables on the “Annual Economics” tab. Column F tells the spreadsheet if the costs need to be converted to a Declining Revenue Requirement (“DRR”) or if they should be treated as expenses.

“Cost DRR Calculator” Tab – This tab looks at cost inputs on the “Cost Input” tab and converts them to DRR as applicable. This tab pulls the useful life assumption from the “Study” tab and pulls the modified accelerated cost recovery system (“MACRS”) assumption from the “Assumptions” tab using lookups.

“Energy” Tab – This tab allows the user to input energy values. These values are outputs from AURORA files.

“Transmission” Tab – This tab allows the user to input transmission projects, costs, and assumed in-service years to calculate transmission costs associated with each study.

“GenericFixedCost” Tab – The “GenericFixedCost” tab has all inputs from the Generic Technologies modeled in the Company’s Technical Appendix Volume 2, Section 1, Technology Screening & Application Standards. This tab also looks up the FT rate input on the “Study” tab for the generic CC to calculate a net present value (“NPV”) for the FT stream associated with that rate. The “Capacity” tab pulls the economic carrying cost (“ECC”) stream in row 6 to be used as the marginal cost of a CC, and the “Cost Inputs” tab pulls the technology costs in rows 9-14 so that the CC modeled as the immediate replacement and the ECC used as the Term Equalization for the deferred replacement are utilizing the same cost assumptions.

“GenericProdCost” Tab – This tab allows the user to input energy values for generic replacement technologies from AURORA. The “Capacity” tab pulls the values to be used as the net marginal cost of a CC, and the “Energy” tab pulls the energy values so that the CC modeled as the immediate replacement and the ECC used as the Term Equalization for the deferred replacement are utilizing the same energy assumptions.

“Annual Economics” Tab – This tab summarizes all costs and benefits determined in the sheets mentioned above. This tab also calculates the NPV associated with each cost category or benefit category.

“EUS” and “GUS” Tabs – These tabs pull the NPVs from the “Annual Economics” tab to summarize all costs and benefits associated with each study or with each generic replacement option.

“Results_TotalCosts” - These tabs allow you to pull specific studies to compare. The ELG pathways are compared on one tab and the 111 pathways are compared on another tab.

“Results_Matrix” Tabs – These tabs pulls the total cost from the “EUS” tab and nets the total cost from the “GUS” tab of the studies the user specifies to create a matrix of benefits among all scenarios.

- Please see “SAM (‘24) - 7.1.7 - Amortization AV Tool TRADE SECRET.xlsm” provided to Commission Staff in the 2025 IRP Workpapers 2-26-25 TRADE SECRET, Technical Appendix Vol 1 TRADE SECRET workpapers folder.

- In the Unit Retirement Study, DRRs are used to represent capital costs for the units being studied. These costs include maintenance capital streams, incremental environmental capital, plant lateral capital, and unit conversion capital. DRRs are also used for the generic replacement unit, or the Generic natural gas combined cycle (“NGCC”) with a 2029 In-Service date. In the Unit Retirement Study, ECCs are used for term equalization marginal cost of a CC capacity calculations. The generic replacement CC and the marginal cost of a CC are derived from the same DRRs and are therefore consistent.

- The Company does not typically use the Format Stale Functions option in excel. The feature was turned on as a result of a recent IT update, and the Company recommends it be turned off manually.

- Please see the Company’s response to STF-JKA-1-21.

- DRRs were calculated in the AV_Tool on the “Cost DRR Calculator” tab using amortization assumptions referenced in subpart (b). AV_Tools for Plants Bowen, Scherer, and Gaston were provided to Commission Staff on January 31, 2025, in the 2025 IRP Workpapers 1-31-25 TRADE SECRET, Technical Appendix Vol 1 TRADE SECRET, Unit Retirement Study TRADE SECRET workpapers folder. The Company did not develop SAMLite files for each cost estimate.

- The AV_Tool allows the user to input transmission costs on the “Cost Inputs” tab or on the “Transmission” tab. Due to the generic nature of the transmission assumption, the transmission cost is modeled in the “Transmission” tab and converted to a 40-year ECC. These cost streams are meant to represent advancement or deferral of transmission investments required for new generation.

- Please see workpapers provided to Commission Staff on January 31, 2025, in the 2025 IRP Workpapers 1-31-25 TRADE SECRET, Technical Appendix Vol 1 TRADE SECRET, Unit Retirement Study TRADE SECRET, Aurora Output Files TRADE SECRET folders. The methodology is consistent with the methodology described in Technical Appendix Volume 1, Section 4, Unit Retirement Study.