STF-JKA-2-25

Question:

Regarding the Nuclear Unit Upgrades,

        - Provide any additional documentation the Company has that explains the Extended Power Uprates, and the Maximum Extended Load Line Limit enhancements

        - What companies will be helping the Company perform these uprates?

        - What other nuclear units is the Company aware of that have either performed these upgrades or have decided to perform these upgrades similar to the Company?  When were the upgrades performed?

Response:

- Utilities have been using power uprates since the 1970s as a way to increase the power output of nuclear plants. To increase the power output of a reactor, typically more highly enriched uranium fuel and/or more fresh fuel is used. This enables the reactor to produce more thermal energy and more steam, thereby driving a turbine generator to produce more electricity. In order to accomplish this, components such as pipes, valves, pumps, heat exchangers, electrical transformers and generators, must be able to accommodate the conditions that would exist at the higher power level. For example, a higher power level usually involves higher steam and water flow through the systems used in converting the thermal power into electric power. These systems must be capable of accommodating the higher flows.

In some instances, licensees will modify and/or replace components in order to accommodate a higher power level. Depending on the desired increase in power level and original equipment design, this can involve major and costly modifications to the plant such as the replacement or retrofit of main turbines and/or main generators. All of these factors must be analyzed by the licensee as part of a request for a power uprate, which is accomplished by amending the plant's operating license. The analyses must demonstrate that the proposed new configuration remains safe and that measures continue to be in place to protect the health and safety of the public. These analyses, which span many technical disciplines and may be complex, are reviewed by the Nuclear Regulatory Commission’s (“NRC”) technical and legal staffs and NRC management before a request for a power uprate is approved.

Maximum Extended Load Line Limit Plus (“MELLLA+”) is an approved method used in a boiling water reactor (“BWR”) power plant to safely increase the allowed core flow range. A plant is licensed to operate within a specified power and flow domain. By giving operators a larger range of licensed core flow, MELLLA+ allows more operational flexibility and can improve plant capacity. MELLLA+ is increasingly important for BWRs with Extended Power Uprates (“EPU”) since EPU by itself reduces the allowed core flow range. MELLLA+ restores the allowed range of core flow, returning operational flexibility to the operators.

- Westinghouse, General Electric Vernova, General Electric Steam, Sargent & Lundy, Enercon, Southern Company Services, Day and Zimmerman are the major suppliers, vendors, and engineering firms that will be assisting Southern Nuclear with the uprates.

- The NRC website link shows the numerous uprates that have been performed in the United States, the type of upgrade and when it was performed: