STF-JKA-2-40

Question:

See p. 41 of the Main Document. Why was it considered reasonable not to round the short-term Reserve Margin Targets, but it was considered important to round the Long-term Reserve Margin Target.

Response:

The results of the long-term reserve margin analysis resulted in a winter target reserve margin (“TRM”) of 26.00% and a summer TRM of 20.00%. The 1:10 loss of load expectation (“LOLE”) threshold for winter was 25.75%. However, the risk-adjusted economic optimal reserve margin (“EORM”) confidence intervals continued to support the current 26.00% recommendation. A 26.00% Winter TRM is also consistent with results from the 2018 and 2021 Reserve Margin Studies and maintaining this TRM provides stability to the integrated resource planning process. See Figure 3 on page (viii) of the 2024 Reserve Margin Study provided in Technical Appendix Volume 1 for the winter season confidence intervals considered.

The minimum acceptable summer TRM was 19.55% based on the annual 1:10 LOLE threshold. However, none of the risk-adjusted EORM confidence intervals supported a TRM below 20.00%. See Figure 4 on page (ix) of the 2024 Reserve Margin Study provided in Technical Appendix Volume 1 for the summer season confidence intervals considered.

The short-term analysis supported an adjustment of 0.50%, which is consistent with the 2018 and 2022 Reserve Margin Studies. Maintaining the current short-term reserve margin adjustment provides stability to the integrated resource planning process.