HR-2-2

Information requested by:  Walmart – Stephanie Eaton

Question:

Q.	I’d like to make a Hearing Request for any additional information that GPC could share to understand the test that was done last week at Plant McDonough as it relates to the press release published by the Company.

Response:

As part of a Research & Design project, Georgia Power and Mitsubishi Power collaborated and performed a series of hydrogen co-firing demonstration tests at the Plant McDonough-Atkinson facility between May and June 2025. This is the first validation project of 50% hydrogen co-firing by volume on an advanced class gas turbine in the world. During the month-long test period, seven short duration tests burning between 5% and 50% hydrogen were performed to test the safety and capability of the equipment to use hydrogen as a fuel source with an expected carbon dioxide (“CO2”) reduction of approximately 22%. Over 25,000 lb of hydrogen were consumed with a peak flow rate of over 10,000 lb per hour. This test achieved safe and reliable short-term operation of the Mitsubishi gas turbine with 50% hydrogen by volume and showed improved operational flexibility while maintaining emissions compliance.