Per Commission Rule 515-7-5-.07, the EDC shall include the following minimum filing requirements in its Annual USF Facilities Expansion Plan filing:

|  | - A detailed written technical description of the project(s). The technical description will include, but not be limited to, a discussion of the type, size, and length of the proposed facilities and the proposed route and the community and/or company that will be served. The technical description will affirmatively state if a proposed approach main line-extension is new main or an extension or expansion of existing main as defined in 515-7-5-.01(j)(k). There will be a discussion of the type and size of main proposed to be replaced. The technical description will affirmatively state if the EDC is or is not certificated to serve in the counties where the facilities will be constructed. |
| --- | --- |

Lanier County

This project proposes to serve the city workshop and city wellhouse for the city of Lakeland, Georgia.  The city of Lakeland, Georgia requests that both of these facilities desire to install natural gas fired backup generators for use during primary electric service power loss.

For the workshop, AGL proposes to tie-into existing 2” plastic on S Williams Street at the intersection of S Williams Street and E Footman Avenue install approximately 1,120 feet of 2” plastic (MAOP 60 psi) along E Footman Avenue and S Valdosta Road to reach the workshop.  The workshop will also require an approximate 300 foot 1-1/4” plastic service.

For the wellhouse, AGL proposes to tie-into existing 2” plastic (MAOP 60 psi) on N Pine Street and install an approximate 35 foot 1-1/4” plastic service.

Please see attachment MFR-4, Maps of Proposed Corridors Lanier.  AGL is currently certificated to serve this area.

Bartow County

This project proposes to serve an electricity generation waste recycling operation.  The customer is EcoMaterial Technologies and the entity removes residue from waste ponds and uses the residue to produce tires and asphalt binders among other things.  This is an existing AGL customer that is expanding use of natural gas from approximately 216 CFH to 3,500+ CFH by adding a commercial dryer to process the product.  This expansion is expected to result in the addition of three to four incremental jobs to the area.

AGL proposes to upsize approximately 1,625 feet of 2” MHP steel with 4” plastic originating on Old State Highway 113 northeast of the facility to reach the facility.  An approximately 45-foot 1-1/4” plastic service is required to replace an existing 7-foot service to provide sufficient gas supply for the expansion.

Please see Attachment MFR-4 Maps of Proposed Corridors Bartow.  AGL is currently certificated to serve this area.