Cherokee

This project proposes to serve residential homes in Cherokee County. There are nine (9) existing residences requesting natural gas service in this neighborhood. These residences have committed to take natural gas should the Commission approve this project.  These residences currently use LP-gas.  This project appeared in previous USF Annual Plan filings in 2023 and 2024.  This request is for USF funding to cover the anticipated CIAC.

AGL proposes to tie-into existing 4” plastic on Holly Street at the intersection of Holly Street and Ridge Road and install approximately 1,650 feet of 4” plastic (MAOP 60 psi) along Holly Street and approximately 2,580 feet of 2” plastic (MAOP 60 psi) along Gin Lane and AL Cove to serve these homes.  Approximately 2,325 feet of 1-1/4” plastic will be necessary for the individual service lines to the nine residences.

Please see attachment Map of Proposed Corridor Cherokee.  AGL is currently certificated to serve this area.